PROJECT REPORT

FOR THE

UNIWEST HUB

PROPOSED COMMERCIAL

BUILDING PROJECT

AT

KH-113M,

MIRJAPUR

PARGANA DANKAUR,

TH-GAUTAM

BUDDH

NAGAR NAGAR,

DISTT-

GAUTAM

BUDDH NAGAR

|  |  | PROJECT REPORT |  |
| --- | --- | --- | --- |
|  |  | FOR THE |  |
|  |  | UNIWEST HUB |  |
|  |  | PROPOSED COMMERCIAL |  |
|  | BUILDING PROJECT |  |  |
| AT |  |  |  |
|  |  | KH-113M, MIRJAPUR |  |
|  |  | PARGANA DANKAUR, |  |
|  |  | TH-GAUTAM BUDDH |  |
|  | NAGAR NAGAR, |  |  |
|  |  | DISTT- GAUTAM |  |
|  | BUDDH NAGAR |  |  |

INDEX

CHAPTER 1: PROJECT DESCRIPTION

1.

1 Introduction

1.2

Contact Information

1.3

Project Location

1.4

Pre-Project Appraisal

1.5

Projected Cost of the Project

1.6

Land Use Pattern for the Proposed Project

1.7

Area Statement for the Proposed Project

1.8

Floor Wise Activity Breakup

1.9

Car Parking Statement

1.10

Resource and Construction Material Requirements

1.11

Landscaping

CHAPTER 2: WATER ENVIRONMENT

2.1

Source of Water Utilization

2.2

Water Consumption of the Proposed Project

2.3

Quality and Portability of Available Water Source

2.4

Waste Water Discharge Details

2.5

Water Saving Practices & Methods

2.6

Green Concepts Proposed in the Project

2.7

Design Details of the Proposed Treatment Plant

CHAPTER 3: SOLID WASTE MANAGEMENT

3.1

Construction Phase

3.2

Occupancy Phase

|  | INDEX |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | CHAPTER 1: PROJECT DESCRIPTION |  |
|  | 1. 1 Introduction |  |  |  |
|  |  | 1.2 Contact Information |  |  |
|  | 1.3 Project Location |  |  |  |
|  |  | 1.4 Pre-Project Appraisal |  |  |
|  |  | 1.5 Projected Cost of the Project |  |  |
|  |  |  | 1.6 Land Use Pattern for the Proposed Project |  |
|  |  |  | 1.7 Area Statement for the Proposed Project |  |
|  |  | 1.8 Floor Wise Activity Breakup |  |  |
|  |  | 1.9 Car Parking Statement |  |  |
|  |  |  |  | 1.10 Resource and Construction Material Requirements |
|  | 1.11 Landscaping |  |  |  |
|  |  |  | CHAPTER 2: WATER ENVIRONMENT |  |
|  |  | 2.1 Source of Water Utilization |  |  |
|  |  |  | 2.2 Water Consumption of the Proposed Project |  |
|  |  |  |  | 2.3 Quality and Portability of Available Water Source |
|  |  | 2.4 Waste Water Discharge Details | 2.4 Waste Water Discharge Details |  |
|  |  |  | 2.5 Water Saving Practices & Methods |  |
|  |  |  | 2.6 Green Concepts Proposed in the Project |  |
|  |  |  |  | 2.7 Design Details of the Proposed Treatment Plant |
|  |  |  |  | CHAPTER 3: SOLID WASTE MANAGEMENT |
|  | 3.1 Construction Phase | 3.1 Construction Phase |  |  |
|  | 3.2 Occupancy Phase | 3.2 Occupancy Phase |  |  |

3.2.1

Secondary Sludge from STP

3.2.2

Hazardous Waste

3.2.3

Bio Medical Waste

3.2.4

e-Waste

3.3

Disposal of Excavated Earth and Construction Debris

3.3.1

Excavated Earth Disposal

3.3.2

Construction Debris

CHATER 4: AIR AND NOISE POLLUTION MANAGEMENT

4.1

Air Pollution Source and its Management

4.2

Noise Generation Sources

CHAPTER

5:

RAIN

WATER

HARVESTING&

STORM

MANAGEMENT

5.1

Rain Water harvesting and Storm Water Management

5.2

Volume of Rain Water Harvested

5.3

Storm Management

CHAPTER 6: ENVIRONMENTAL MANAGEMENT PLAN

6.1

Environmental Management Plan

6.2

EMP during Construction Phase

6.2.1

Personnel Safety System

6.2.2

Financial Allocation and Budgetary Provision for EMP

Aspects (construction aspects)

6.3

EMP during Operation Phase

6.3.1

Air Quality Management

6.3.2

Water Quality Management

6.3.3

Noise Management

|  |  |  | 3.2.1 Secondary Sludge from STP |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3.2.2 Hazardous Waste |  |  |  |  |  |
|  |  | 3.2.3 Bio Medical Waste |  |  |  |  |  |
|  |  | 3.2.4 e-Waste |  |  |  |  |  |
|  |  |  |  | 3.3 Disposal of Excavated Earth and Construction Debris |  |  |  |
|  |  |  | 3.3.1 Excavated Earth Disposal |  |  |  |  |
|  |  | 3.3.2 Construction Debris |  |  |  |  |  |
|  |  |  |  | CHATER 4: AIR AND NOISE POLLUTION MANAGEMENT |  |  |  |
|  |  |  | 4.1 Air Pollution Source and its Management |  |  |  |  |
|  |  | 4.2 Noise Generation Sources |  |  |  |  |  |
|  |  |  |  | CHAPTER 5: RAIN WATER HARVESTING& STORM |  |  |  |
|  | MANAGEMENT |  |  |  |  |  |  |
|  |  |  |  | 5.1 Rain Water harvesting and Storm Water Management |  |  |  |
|  |  | 5.2 Volume of Rain Water Harvested |  |  |  |  |  |
|  | 5.3 Storm Management |  |  |  |  |  |  |
|  |  |  |  | CHAPTER 6: ENVIRONMENTAL MANAGEMENT PLAN |  |  |  |
|  |  | 6.1 Environmental Management Plan |  |  |  |  |  |
|  |  | 6.2 EMP during Construction Phase |  |  |  |  |  |
|  |  |  | 6.2.1 Personnel Safety System |  |  |  |  |
|  |  |  |  |  | 6.2.2 Financial Allocation and Budgetary Provision for EMP |  |  |
|  |  |  | Aspects (construction aspects) | Aspects (construction aspects) |  |  |  |
|  |  | 6.3 EMP during Operation Phase |  |  |  |  |  |
|  |  |  | 6.3.1 Air Quality Management |  |  |  |  |
|  |  |  | 6.3.2 Water Quality Management |  |  |  |  |
|  |  | 6.3.3 Noise Management |  |  |  |  |  |

6.3.4

Solid Waste Management

6.4

Health Risk and Disaster Management

6.4.1

Public and Health Safety

6.4.2

Risk and Disaster Management Plan

6.5

EMP Implementation Schedule

6.6

Environmental Monitoring Routines

6.7

Traffic Management Measures

6.8

Energy Conservation Aspects Proposed in the Project

|  |  | 6.3.4 Solid Waste Management |  |
| --- | --- | --- | --- |
|  |  | 6.4 Health Risk and Disaster Management |  |
|  |  | 6.4.1 Public and Health Safety |  |
|  |  |  | 6.4.2 Risk and Disaster Management Plan |
|  |  | 6.5 EMP Implementation Schedule |  |
|  |  | 6.6 Environmental Monitoring Routines |  |
|  |  | 6.7 Traffic Management Measures |  |
|  |  |  | 6.8 Energy Conservation Aspects Proposed in the Project |

CHAPTER 1: PROJECT DESCRIPTION

Project in a Nut Shell

Project Name

Commercial Building

Location

UNIWEST HUB

KH

-

113

M,MIRZAPUR

PARGANA

DANKAUR,GAUTAM BUDDH NAGAR

TH

-

G.B

NAGAR,

DISTT

-

G.B.NAGAR

Total Plot Area

2000 Sqm

Total buildup area

7365 Sq.m

Total cost of project

No. of floors proposed

Project

consists

of

commercial

building with Two basement floor and

Eight floors.

Car parking Details

Total No. of car parking space

proposed is Nos. 114

Water supply

Water supply from

Bore well in the premises

Sanitation

Underground sanitary system facility

for conveying the waste water to the

proposed sewage treatment plant.

Solid water management

Air pollution/noise pollution

generation source

Collection and segregation at source

of generation and the organic waste

will be treated in organic converter

and the inorganic waste will be sent

for recycling 2 No. 500KVA DG set

with acoustics are proposed.

1.1:

Introduction

:

Uniwest Infratech is a premier real estate company based in Greater Noida,

renowned for its commitment to quality and innovation in the property market.

With a diverse portfolio of residential, commercial, and mixed-use developments,

Uniwest Infratech focuses on creating sustainable and aesthetically pleasing

spaces. The company prioritizes customer satisfaction, ensuring that each project

meets the highest standards of design and construction. Leveraging advanced

technologies and a customer-centric approach, Uniwest Infratech has established

itself as a trusted name in the industry. Its strategic location in Greater Noida

allows it to capitalize on the region's rapid growth, offering modern and

convenient living and working environments to its clients.

|  |  |  |  | CHAPTER 1: PROJECT DESCRIPTION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Project Name Commercial Building |  |  |  |  |  |  |
|  |  |  | Location UNIWEST HUB |  |  |  |  |  |  |
|  |  |  |  |  |  |  | KH - 113 M,MIRZAPUR PARGANA |  |  |
|  |  |  |  |  |  | DANKAUR,GAUTAM BUDDH NAGAR |  |  |  |
|  |  |  |  | Total Plot Area TH 2000 Sqm - G.B NAGAR, DISTT - |  |  |  |  |  |
|  |  |  | Total buildup area G.B.NAGAR 7365 Sq.m |  |  |  |  |  |  |
|  | Total cost of project |  |  |  |  |  |  |  |  |
|  |  |  |  | No. of floors proposed Project consists of commercial |  |  |  |  |  |
|  |  |  |  |  |  |  | building with Two basement floor and |  |  |
|  |  |  |  |  | Eight floors. |  |  |  |  |
|  |  |  |  |  |  | proposed is Nos. 114 |  |  |  |
|  |  |  | Water supply Water supply from |  |  |  |  |  |  |
|  |  |  |  |  |  | Bore well in the premises |  |  |  |
|  |  |  |  | Sanitation Underground sanitary system facility |  |  |  |  |  |
|  |  |  |  |  |  |  | for conveying the waste water to the |  |  |
|  |  |  |  |  |  | proposed sewage treatment plant. |  |  |  |
|  |  |  |  | Solid water management Collection and segregation at source |  |  |  |  |  |
|  |  |  |  | Air pollution/noise pollution and the inorganic waste will be sent |  |  |  |  |  |
|  |  |  |  | generation source for recycling 2 No. 500KVA DG set |  |  |  |  |  |
|  |  |  |  |  |  |  | of generation and the organic waste |  |  |
|  |  |  |  |  |  |  | will be treated in organic converter |  |  |
|  |  |  |  |  |  | with acoustics are proposed. |  |  |  |
|  | 1.1: Introduction : |  |  |  |  |  |  |  |  |
|  |  |  |  | Uniwest Infratech is a premier real estate company based in Greater Noida, |  |  |  |  |  |
|  |  |  |  | renowned for its commitment to quality and innovation in the property market. |  |  |  |  |  |
|  |  |  |  | With a diverse portfolio of residential, commercial, and mixed-use developments, |  |  |  |  |  |
|  |  |  |  | Uniwest Infratech focuses on creating sustainable and aesthetically pleasing |  |  |  |  |  |
|  |  |  |  | spaces. The company prioritizes customer satisfaction, ensuring that each project |  |  |  |  |  |
|  |  |  |  | meets the highest standards of design and construction. Leveraging advanced |  |  |  |  |  |
|  |  |  |  | technologies and a customer-centric approach, Uniwest Infratech has established |  |  |  |  |  |
|  |  |  |  | itself as a trusted name in the industry. Its strategic location in Greater Noida |  |  |  |  |  |
|  |  |  |  | allows it to capitalize on the region's rapid growth, offering modern and |  |  |  |  |  |
|  |  |  | convenient living and working environments to its clients. |  |  |  |  |  |  |

1.2

: Contact Information

:

Contact information of the project proponent is as below:

Sl. No

.

Description

Information

1

Proponent

UNIWEST INFRATECH Pvt. Ltd.

2

Address

6th floor, office no-606, kasana tower,

alpha-1st,

commercial

belt

greater

noida

3

Telephone No.

9811335758

4

e-mail ID

uniwestinfratech2023@gmail.com

5

Website

https://uniwestarcade.com/

1.3

: Project Location:

The Site is located KH-113M,MIRJAPUR PARGANA DANKAUR TH-

GAUTAM BUDDH NAGAR,DISTT- GAUTAM BUDDH NAGAR between

Latitude 28°18'18.5"N 77°33'19.3"E and

located at a distance of about

6.00 km from Ernakulam north railway station.

The general topographical features of the area reveal that the

proposed

project site and its surroundings is generally a plain land with gentle slope.

The soil nature observed is generally alluvial soil.

The land around the

project

site

indicates

a

mixed

land

use

pattern. There

are

residential

Apartments, villas, and commercial Establishments surrounded by areas like

Supertech Aerocaity, Gaur Galleria etc. The project site is a vacant land.

Transportation

:

The project site is located at 290M from GYC Exit point

at Yamuna Expressway.

Agricultural Lands

:

Mixed land with agricultural lands are seen in the

surrounding area.

Airport

: Jewar International Airport at 20Km

Water Bodies

:

Yamuna river is at 12.00Km from project site.

|  |  | 1.2 : Contact Information : |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Contact information of the project proponent is as below: |  |  |  |  |  |
|  |  |  | Sl. No . Description Information | Sl. No . Description Information |  |  |  |  |  |
|  |  |  |  |  | 1 Proponent UNIWEST INFRATECH Pvt. Ltd. |  |  |  |  |
|  |  |  |  |  | 2 Address 6th floor, office no-606, kasana tower, |  |  |  |  |
|  |  |  | 3 Telephone No. noida 9811335758 |  |  |  |  |  |  |
|  |  |  |  |  | 4 e-mail ID uniwestinfratech2023@gmail.com |  |  |  |  |
|  |  |  |  | 5 Website https://uniwestarcade.com/ |  |  |  |  |  |
|  |  | 1.3 : Project Location: |  |  |  |  |  |  |  |
|  |  |  |  |  | The Site is located KH-113M,MIRJAPUR PARGANA DANKAUR TH- |  |  |  |  |
|  |  |  |  |  | GAUTAM BUDDH NAGAR,DISTT- GAUTAM BUDDH NAGAR between |  |  |  |  |
|  |  |  |  |  | Latitude 28°18'18.5"N 77°33'19.3"E and located at a distance of about |  |  |  |  |
|  |  |  | 6.00 km from Ernakulam north railway station. | 6.00 km from Ernakulam north railway station. |  |  |  |  |  |
|  |  |  |  |  | The general topographical features of the area reveal that the proposed |  |  |  |  |
|  |  |  |  |  | project site and its surroundings is generally a plain land with gentle slope. |  |  |  |  |
|  |  |  |  |  | The soil nature observed is generally alluvial soil. The land around the |  |  |  |  |
|  |  |  |  |  | project site indicates a mixed land use pattern. There are residential |  |  |  |  |
|  |  |  |  |  | Apartments, villas, and commercial Establishments surrounded by areas like |  |  |  |  |
|  |  |  |  |  | Supertech Aerocaity, Gaur Galleria etc. The project site is a vacant land. |  |  |  |  |
|  |  |  |  |  | Transportation : The project site is located at 290M from GYC Exit point | Transportation : The project site is located at 290M from GYC Exit point |  |  |  |
|  |  | at Yamuna Expressway. |  |  |  |  |  |  |  |
|  |  |  |  |  | Agricultural Lands : Mixed land with agricultural lands are seen in the |  |  |  |  |
|  | surrounding area. |  |  |  |  |  |  |  |  |
|  |  |  |  | Airport : Jewar International Airport at 20Km |  |  |  |  |  |
|  |  |  |  |  | Water Bodies : Yamuna river is at 12.00Km from project site. |  |  |  |  |

Sl. No.

Description

Bearings with respect to site

1

Adjacent properties

APEX AEROCITY

NH Bypass

GYC GALLERIAA

South

North

West

East

Location Map

1.4

4: Pre-Project Appraisal:

Any activity involving construction and operation is expected to cause

impacts

on

surrounding

environment.

The

impact

may

be

adverse

or

beneficial, short term or long term and reversible or irreversible.

SITE

LOCATION

|  |  |  | Sl. No. Description Bearings with respect to site |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 Adjacent properties |  |  |  |  |  |  |
|  |  |  | NH Bypass West |  |  |  |  |  |
|  |  |  |  | North |  |  |  |  |
|  | Location Map |  |  |  |  |  |  |  |
|  |  | 1.4 4: Pre-Project Appraisal: |  |  |  |  |  |  |
|  |  |  |  | Any activity involving construction and operation is expected to cause |  |  |  |  |
|  |  |  |  | impacts on surrounding environment. The impact may be adverse or |  |  |  |  |
|  |  |  | beneficial, short term or long term and reversible or irreversible. |  |  |  |  |  |
|  | SITE |  |  |  |  |  |  |  |
|  | LOCATION |  |  |  |  |  |  |  |

As per the EIA Notification issued by MOEF, Govt. of India, the

project d o e s n o t

needs to obtain consent from the state pollution Board

and

Environmental

Clearance

(EC)

from

the

State

Level

Environment

Impact Assessment Authority (SEIAA) The Built-up area of the project is

5000 Sq.M. which doesnot comes under serial No.8(a) of Schedule, EIA

2006, Notification

(Building

and

Construction

Projects

with

a

built

up

area

>1,50,000Sq.M. and < 3,00,000 Sq.M.

Hence Environmental Clearance is n o t sought.

1.5

: Projected Cost of the Project

:

The total projected cost of the project is presented as below:

Sl. No.

Particulars

Cost of Rupees.

1

Land cost (BOOK VALUE)

11635000

2

LAND Cost (STAMP VALUE)

814500

3

Construction cost @ rate of

Rs. 3 000.00Sq.Ft. of built-up

area = 7365 sq.m

Rs. 22095000

4

Miscellenous works and outer

development of the Project

1,0000000

1.6

6: Land Use Pattern for the Proposed Project:

Sl.

No.

Particulars

Area in

Sq.M.

Area in Acres

Percentage

1

Total plot Area

2000

0.494

2

Road Widening

---

---

---

|  |  |  |  |  |  | As per the EIA Notification issued by MOEF, Govt. of India, the |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | project d o e s n o t needs to obtain consent from the state pollution Board |  |  |  |  |
|  |  |  |  |  |  | and Environmental Clearance (EC) from the State Level Environment |  |  |  |  |
|  |  |  |  |  |  | Impact Assessment Authority (SEIAA) The Built-up area of the project is |  |  |  |  |
|  |  |  |  |  |  | 5000 Sq.M. which doesnot comes under serial No.8(a) of Schedule, EIA |  |  |  |  |
|  |  |  |  |  |  | 2006, Notification (Building and Construction Projects with a built up |  |  |  |  |
|  | area |  |  |  |  |  |  |  |  |  |
|  |  |  | >1,50,000Sq.M. and < 3,00,000 Sq.M. |  |  |  |  |  |  |  |
|  |  |  |  | Hence Environmental Clearance is n o t sought. |  |  |  |  |  |  |
|  |  |  | 1.5 : Projected Cost of the Project : |  |  |  |  |  |  |  |
|  |  |  |  |  | The total projected cost of the project is presented as below: |  |  |  |  |  |
|  |  |  |  |  | Sl. No. Particulars Cost of Rupees. |  |  |  |  |  |
|  |  |  |  | 1 Land cost (BOOK VALUE) 11635000 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | LAND Cost (STAMP VALUE) 814500 |  |  |  |  |  |
|  |  |  | 3 Rs. 3 000.00Sq.Ft. of built-up |  |  |  |  |  |  |  |
|  |  |  |  |  | Construction cost @ rate of Rs. 22095000 |  |  |  |  |  |
|  |  |  | area = 7365 sq.m |  |  |  |  |  |  |  |
|  |  |  |  | 4 Miscellenous works and outer 1,0000000 |  |  |  |  |  |  |
|  |  |  |  | development of the Project |  |  |  |  |  |  |
|  |  |  |  | 1.6 6: Land Use Pattern for the Proposed Project: |  |  |  |  |  |  |
|  |  |  |  |  | Sl. Particulars Area in Area in Acres Percentage |  |  |  |  |  |
|  |  | No. Sq.M. | No. Sq.M. |  |  |  |  |  |  |  |
|  |  |  | 1 Total plot Area 2000 0.494 |  |  |  |  |  |  |  |
|  |  |  |  |  | 2 Road Widening --- --- --- |  |  |  |  |  |

3

Ground

coverage area

725

0.18

36

1.7

7: Area Statement for the Proposed Project

:

|  |  | 3 Ground 725 0.18 36 |  |  |
| --- | --- | --- | --- | --- |
|  | coverage area |  |  |  |
|  |  | 1.7 7: Area Statement for the Proposed Project : |  |  |

1.8

8: Floor Wise Activity Breakup:

Sl. No.

Floor

Activity

1

Basement1

Car parking, MEP services, Fire

Fighting equipments Room

2

Basement2

Car parking, MEP services, Fire

Fighting equipments Room

3

Lower Ground Floor

Retail Shop

4

Upper Ground Floor

“

“

5

First Floor

“

“

6

Second Floor

Office

7

Third Floor

Studio

8

Fourth Floor

Studio

9

10

Fifth Floor

Studio

10

Sixth Floor

Studio

11

Terrace Floor

|  |  | 1.8 8: Floor Wise Activity Breakup: |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fighting equipments Room | Fighting equipments Room |
|  |  |  |  | Fighting equipments Room | Fighting equipments Room |
|  |  | 6 Second Floor Office |  |  |  |
|  |  | 7 Third Floor Studio |  |  |  |
|  |  | 8 Fourth Floor Studio |  |  |  |
|  |  | 9 Fifth Floor Studio |  |  |  |
|  |  | 10 10 Sixth Floor Studio |  |  |  |
|  | 11 Terrace Floor |  |  |  |  |

1.90: Car Parking Statement:

Sl. No.

Description

Car parking

1

Basement 1+2

86 Nos

3

Building premises

10 Nos

1.10

0: Resource and Construction Material Requirements:

Construction of the proposed building will require the use of renewable

and non renewable resources including wood, gravel, sand, steel, concrete,

cement and concrete blocks. The materials will be used during construction

period only and will not be required during the operation phase of the project.

The estimated quantities of major construction materials required are given in

table below.

Fuel such as diesel and oil will be used during the construction and

operation of the project for mechanical and electrical equipments.

Electricity

will

be

used

during

construction

to

provide

power

to

construction

equipment

and

during

operation

for lighting

of

building

and

running utility equipment.

Estimated Quantities of construction Materials

:

Sl. No.

Description

Unit

Quantity

1

Reinforced steel

Metric ton

350

2

Concrete

cum

3000

|  |  |  | 1.90: Car Parking Statement: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Sl. No. Description Car parking |  |  |  |  |  |  |
|  |  |  |  | 1 Basement 1+2 86 Nos |  |  |  |  |  |  |  |
|  |  |  |  | 3 Building premises 10 Nos |  |  |  |  |  |  |  |
|  |  |  |  |  | 1.10 0: Resource and Construction Material Requirements: | 1.10 0: Resource and Construction Material Requirements: |  |  |  |  |  |
|  |  |  |  |  |  | Construction of the proposed building will require the use of renewable |  |  |  |  |  |
|  |  |  |  |  |  | and non renewable resources including wood, gravel, sand, steel, concrete, |  |  |  |  |  |
|  |  |  |  |  |  | cement and concrete blocks. The materials will be used during construction |  |  |  |  |  |
|  |  |  |  |  |  | period only and will not be required during the operation phase of the project. |  |  |  |  |  |
|  |  |  |  |  |  | The estimated quantities of major construction materials required are given in |  |  |  |  |  |
|  | table below. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Fuel such as diesel and oil will be used during the construction and |  |  |  |  |  |
|  |  |  |  |  | operation of the project for mechanical and electrical equipments. |  |  |  |  |  |  |
|  |  |  |  |  |  | Electricity will be used during construction to provide power to |  |  |  |  |  |
|  |  |  |  |  |  | construction equipment and during operation for lighting of building and |  |  |  |  |  |
|  |  | running utility equipment. |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Estimated Quantities of construction Materials : |  |  |  |  |  |  |  |
|  |  |  |  |  | Sl. No. Description Unit Quantity |  |  |  |  |  |  |
|  |  |  |  |  | 1 Reinforced steel Metric ton 350 |  |  |  |  |  |  |
|  |  |  |  |  | 2 Concrete cum 3000 |  |  |  |  |  |  |

1.11

1: Landscaping:

The landscaping of project has been planned to provide a clean, healthy

and beautiful green environment for the people within the proposed project site.

In

the

present

project

sufficient

space

has

been

designated

for

greenery

development.

|  | 1.11 1: Landscaping: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | and beautiful green environment for the people within the proposed project site. |  |  |  |  |
|  |  |  |  | In the present project sufficient space has been designated for greenery |  |  |  |  |

CHAPTER 2: WATER ENVIRONMENT

2.1

1: Source of Water Utilization:

The most important aspect under the water supply scheme is the selection

of source of water, which should be reliable and potable.

The project being a commercial development with office space and retail

activities, water demand is for domestic purpose, public uses and firefighting

purposes. It includes the quantity of water required for drinking, flushing, land

scaping etc. The quantity of water required depends on the habits, social status

and climatic conditions. In India on an average the water requirement for

commercial establishment is 45 LPCD. The anticipated water demand is worked

out by taking into consideration that the project is fully developed.

2.2

2: Water Consumption of the Proposed Project:

Sl. No.

Occupants

Area

excluding car

parking

Total No. of

occupants

considering 1

person per 10

sq.m

Water

required

considering

45 LPCD

1

Occupants

5000 M

2

500

22,500

2

Visitors per day

--

1000 Nos.

15000

(15Ltrper

visitor)

3

Total

water

consumption

per day

37,500 Ltrs.

From the above demand, about 80% of water converts into sewage which

amounts to 30000 Ltrs. Hence a sewage treatment plant of 30 KLD is proposed.

2.3

3: Quality & Portability of Available Water Source:

The quality of water from the source of origin is good and only disinfection

is envisaged. After the treatment of the water, it is distributed through a net

work of distribution system.

The distribution system consists of pipes of

|  |  |  |  | CHAPTER 2: WATER ENVIRONMENT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2.1 1: Source of Water Utilization: |  |  |  |  |  |
|  |  |  | of source of water, which should be reliable and potable. |  |  |  |  |
|  |  |  |  | activities, water demand is for domestic purpose, public uses and firefighting |  |  |  |
|  |  |  |  | purposes. It includes the quantity of water required for drinking, flushing, land |  |  |  |
|  |  |  |  | scaping etc. The quantity of water required depends on the habits, social status |  |  |  |
|  |  |  |  | and climatic conditions. In India on an average the water requirement for |  |  |  |
|  |  |  |  | commercial establishment is 45 LPCD. The anticipated water demand is worked |  |  |  |
|  |  |  | out by taking into consideration that the project is fully developed. |  |  |  |  |
|  |  |  | 2.2 2: Water Consumption of the Proposed Project: |  |  |  |  |
|  |  |  |  | Sl. No. Occupants Area Total No. of Water |  |  |  |
|  |  |  |  |  | excluding car occupants required | excluding car occupants required |  |
|  |  |  |  |  |  | parking considering 1 considering |  |
|  |  |  |  |  | sq.m |  |  |
|  |  |  |  | 1 Occupants 5000 M 500 22,500 |  |  |  |
|  |  |  | 2 Visitors per day -- 1000 Nos. 15000 | 2 Visitors per day -- 1000 Nos. 15000 |  |  |  |
|  |  |  |  |  |  |  | (15Ltrper |
|  |  |  |  |  |  |  | visitor) |
|  |  |  |  | 3 Total water 37,500 Ltrs. |  |  |  |
|  |  |  |  |  | consumption | consumption |  |
|  |  |  |  |  | per day |  |  |
|  |  |  |  | From the above demand, about 80% of water converts into sewage which |  |  |  |
|  |  |  |  | amounts to 30000 Ltrs. Hence a sewage treatment plant of 30 KLD is proposed. |  |  |  |
|  |  |  |  | 2.3 3: Quality & Portability of Available Water Source: |  |  |  |
|  |  |  |  | is envisaged. After the treatment of the water, it is distributed through a net |  |  |  |
|  |  |  |  | work of distribution system. The distribution system consists of pipes of |  |  |  |

various sizes, valves, meters, pumps and hydrant valves. The valve provided in

the distribution network will control flow of water through the system. Fire

hydrants are provided to connect water to fire-fighting equipment during fire.

2.4

4: Waste Water Discharge Details:

Wastes of different types such as spent water from toilets, water closets,

semi liquid waste of human excreta are produced daily. If proper arrangements

for collection, treatment and disposal of the wastes produced are not made,

unsanitary conditions will develop and it will become impossible for the public

to live. Therefore, it is most essential to collect, treat and dispose all the

sanitary waste produced. Generally 80% of the water supplied comes out as

sewage.

Therefore,

the

total

quantity

of

waste

water

generated

from

the

proposed project is worked out as below:

Total water demand for the proposed

project

37500 Liters.

Waste

water

generated

from

the

project

(assuming

waste

water

generation to be about 80% of the total

water)

30000 Liters

Say 30 KLD

2.5

5: Water Saving Practices & Methods:

Many measures can be adapted to reduced water demand through use of

water efficient practices and devices; these would result in significant saving of

water. Some of the common practices and devices than can save water are

given below.

a)

Water conserving flushing fixtures operating on sensors shall be used for

urinals.

b)

Dual flushing cistern, low flow fixtures and faucets shall be used.

c)

Water closet with low volume flushing cistern/flush valve shall be used.

d)

Water conserving means like pop-up sprinklers, drip irrigation etc shall

be employed to irrigate the land-scape areas.

e)

Stored rain water shall be used for domestic purposes after necessary

treatment.

|  |  |  |  |  | various sizes, valves, meters, pumps and hydrant valves. The valve provided in |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | the distribution network will control flow of water through the system. Fire |  |  |  |
|  |  |  |  | hydrants are provided to connect water to fire-fighting equipment during fire. |  |  |  |  |
|  |  |  | 2.4 4: Waste Water Discharge Details: |  |  |  |  |  |
|  |  |  |  |  | Wastes of different types such as spent water from toilets, water closets, |  |  |  |
|  |  |  |  |  | semi liquid waste of human excreta are produced daily. If proper arrangements |  |  |  |
|  |  |  |  |  | for collection, treatment and disposal of the wastes produced are not made, |  |  |  |
|  |  |  |  |  | unsanitary conditions will develop and it will become impossible for the public |  |  |  |
|  |  |  |  |  | to live. Therefore, it is most essential to collect, treat and dispose all the |  |  |  |
|  |  |  |  |  | sanitary waste produced. Generally 80% of the water supplied comes out as |  |  |  |
|  |  |  |  |  | sewage. Therefore, the total quantity of waste water generated from the |  |  |  |
|  |  | proposed project is worked out as below: |  |  |  |  |  |  |
|  |  |  | Total water demand for the proposed 37500 Liters. |  |  |  |  |  |
| project |  |  |  |  |  |  |  |  |
|  |  |  | Waste water generated from the 30000 Liters |  |  |  |  |  |
|  |  | project (assuming waste water |  |  |  |  |  |  |
|  |  |  | generation to be about 80% of the total Say 30 KLD |  |  |  |  |  |
| water) |  |  |  |  |  |  |  |  |
|  |  |  | 2.5 5: Water Saving Practices & Methods: |  |  |  |  |  |
|  |  |  |  |  | Many measures can be adapted to reduced water demand through use of |  |  |  |
|  |  |  |  |  | water efficient practices and devices; these would result in significant saving of |  |  |  |
|  |  |  |  |  | water. Some of the common practices and devices than can save water are |  |  |  |
|  | given below. |  |  |  |  |  |  |  |
|  |  |  |  |  | a) Water conserving flushing fixtures operating on sensors shall be used for |  |  |  |
|  | urinals. |  |  |  |  |  |  |  |
|  |  |  |  | b) Dual flushing cistern, low flow fixtures and faucets shall be used. |  |  |  |  |
|  |  |  |  |  | c) Water closet with low volume flushing cistern/flush valve shall be used. |  |  |  |
|  |  |  |  |  | d) Water conserving means like pop-up sprinklers, drip irrigation etc shall |  |  |  |
|  |  |  | be employed to irrigate the land-scape areas. |  |  |  |  |  |
|  |  |  |  |  | e) Stored rain water shall be used for domestic purposes after necessary |  |  |  |
|  | treatment. |  |  |  |  |  |  |  |

2.6

6: Green Concepts Proposed In The Project:

The proposed commercial development will be constructed by adopting

the following green concepts.

a)

Dual flushing system & low flow water saving fixtures.

b)

100% reuse of waste water treated on site.

c)

Rain water storage and its reuse on site.

d)

Reducing the impervious cover by using perforated pavement system.

e)

Variable speed fans on AHU, Heat recovery, free cooling and CFC free

efficient chiller and cooling tower.

2.7

7: Design Details of Proposed Treatment Plant:

Design Details of STP:

Total Plinth Area

Total area excluding car parking

5000 Sq.m

5000 Sq.m

Number of occupants @ 1 person/ 10 m2

500

Water consumption @ 45 Liters/head

22,500

Number of visitors per day

1000

Water consumption @ 15 Liters/visitor

15000

Total water consumption in the complex

37,500

Waste water generation (80% of water consumption)

30,000 Liters

Quantity of sewage water (30% of wastewater

quantity)

9000 Liters/Hour

Quantity of sullage water (70% of wastewater

quantity)

21000 Liters/hour

Capacity Proposed = 30KLD

SEWAGE TREATMENT PLANT FOR THE COMMERCIAL COMPLEX

|  |  |  |  | 2.6 6: Green Concepts Proposed In The Project: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | The proposed commercial development will be constructed by adopting |  |  |
|  |  | the following green concepts. |  |  |  |  |  |
|  |  |  |  | a) Dual flushing system & low flow water saving fixtures. |  |  |  |
|  |  |  | b) 100% reuse of waste water treated on site. |  |  |  |  |
|  |  |  | c) Rain water storage and its reuse on site. |  |  |  |  |
|  |  |  |  |  | d) Reducing the impervious cover by using perforated pavement system. |  |  |
|  |  |  |  |  | e) Variable speed fans on AHU, Heat recovery, free cooling and CFC free |  |  |
|  |  |  | efficient chiller and cooling tower. |  |  |  |  |
|  |  |  |  | 2.7 7: Design Details of Proposed Treatment Plant: |  |  |  |
|  | Design Details of STP: |  |  |  |  |  |  |
|  |  |  |  |  | Total Plinth Area 5000 Sq.m |  |  |
|  |  |  |  |  | Total area excluding car parking 5000 Sq.m |  |  |
|  |  |  |  | Number of occupants @ 1 person/ 10 m2 500 |  |  |  |
|  |  |  |  | Water consumption @ 45 Liters/head 22,500 |  |  |  |
|  |  |  |  | Number of visitors per day 1000 |  |  |  |
|  |  |  |  | Water consumption @ 15 Liters/visitor 15000 |  |  |  |
|  |  |  |  | Total water consumption in the complex 37,500 |  |  |  |
|  |  |  |  |  | Waste water generation (80% of water consumption) 30,000 Liters |  |  |
|  |  |  |  |  | Quantity of sewage water (30% of wastewater 9000 Liters/Hour |  |  |
| quantity) |  |  |  |  |  |  |  |
|  |  |  |  |  | Quantity of sullage water (70% of wastewater 21000 Liters/hour |  |  |
| quantity) |  |  |  |  |  |  |  |
|  |  |  |  |  | Capacity Proposed = 30KLD |  |  |
|  |  |  |  |  | SEWAGE TREATMENT PLANT FOR THE COMMERCIAL COMPLEX |  |  |

Raw water Characteristics:

The wastewater is generated in the project is from the toilets, bathing area

and kitchen area. The effluent character is expected as below.

BOD

300 mg/l

COD

540 mg/l

pH

6.5 to 8

Proposed Treatment system:

The

wastewater

treatment

system

suggested

will

have

the

following

components and expected reductions in the pollutants are given below.

Sl. No.

Treatment

System

Remarks

1

Grit

chamber

For the removal of debris and big particles

from

the

effluent-

we

suggest

two

grit

chambers in line. We should provide the

grit

chambers

and

screen

chambers

for

every

wastewater

stream

leading

to

the

collection tank.

2

Oil Trap

For removal of oil and grease from the

effluent. The presence of Oil and grease in

the

effluent

will

affect

subsequent

treatment systems.

3

Septic Tank

The sewage should be collected in a septic

tank and the over flow should be diverted

to

the

collection

sump.

The

septic tank

should be as per IS 2970.

4

Collection

Tank

It is a buffer for the treatment system. For

efficient treatment of all treatment systems,

the flow should be regularized to a uniform

and

constant

flow

rate.

As

the

effluent

generation is fluctuating, the collection tank

will act as a buffer. The overflow

from

septic tank and the waste water from oil trap

should be collected in the collection tank.

5

Anox Tank

The wastewater from the collection tank is

|  | Raw water Characteristics: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | The wastewater is generated in the project is from the toilets, bathing area |  |  |  |  |  |
|  |  |  | and kitchen area. The effluent character is expected as below. |  |  |  |  |  |  |
|  |  |  |  | BOD 300 mg/l |  |  |  |  |  |
|  |  |  |  | COD 540 mg/l |  |  |  |  |  |
|  |  |  |  | pH 6.5 to 8 |  |  |  |  |  |
|  |  |  |  | The wastewater treatment system suggested will have the following |  |  |  |  |  |
|  |  |  |  | components and expected reductions in the pollutants are given below. |  |  |  |  |  |
|  |  | Sl. No. Treatment Remarks |  |  |  |  |  |  |  |
|  |  |  |  | 1 Grit For the removal of debris and big particles |  |  |  |  |  |
|  |  |  |  |  | chamber from the effluent- we suggest two grit |  |  |  |  |
|  |  |  |  |  |  | chambers in line. We should provide the |  |  |  |
|  |  |  |  |  |  | grit chambers and screen chambers for |  |  |  |
|  |  |  |  |  |  | every wastewater stream leading to the |  |  |  |
|  |  |  |  | collection tank. |  |  |  |  |  |
|  |  |  |  | 2 Oil Trap For removal of oil and grease from the |  |  |  |  |  |
|  |  |  |  |  |  | effluent. The presence of Oil and grease in |  |  |  |
|  |  |  |  |  |  | the effluent will affect subsequent |  |  |  |
|  |  |  |  | treatment systems. |  |  |  |  |  |
|  |  |  |  | 3 Septic Tank The sewage should be collected in a septic |  |  |  |  |  |
|  |  |  |  |  |  | tank and the over flow should be diverted |  |  |  |
|  |  |  |  |  |  | to the collection sump. The septic tank |  |  |  |
|  |  |  |  |  | should be as per IS 2970. |  |  |  |  |
|  |  |  |  | 4 Collection It is a buffer for the treatment system. For |  |  |  |  |  |
|  |  |  |  |  | Tank efficient treatment of all treatment systems, |  |  |  |  |
|  |  |  |  |  |  | the flow should be regularized to a uniform |  |  |  |
|  |  |  |  |  |  | and constant flow rate. As the effluent |  |  |  |
|  |  |  |  |  |  | generation is fluctuating, the collection tank |  |  |  |
|  |  |  |  |  |  | will act as a buffer. The overflow from |  |  |  |
|  |  |  |  |  |  | septic tank and the waste water from oil trap |  |  |  |
|  |  |  |  |  |  | should be collected in the collection tank. |  |  |  |
|  |  |  |  | 5 Anox Tank The wastewater from the collection tank is |  |  |  |  |  |

transferred at a uniform rate to the treatment

system. The anox tank is provided to nitrify

the

nitrogen

compounds

present

in

the

wastewater. It is done in the absence of air

and with agitation for better mixing.

6

Aerobic

Treatment -

MBBR

Effluent from the anaerobic treatment will

further

treat

aerobically

using

MBBR

(Moving Bed Bio- Reactor) media. In this

treatment

system, the

aeration tanks

(2

numbers)

is

filled

with

specially

made

(imported)

plastic

body

(called

MBBR

Media) for the aggressive growth of useful

bactria in the tank for the reduction of

biological content. 50% fill of media is

recommened. The removal efficiency is

expected to be about 98%.

7

Settling

Tanks

/

Clarifier

The

effluent

from

MBBR

tanks

will

contain dead cells of bacteria and high

content of suspended solids. To remove

these matters, a secondary settling with

flocculation is suggested. In order to have

a better settling and if required a chemical

settling is provided. We suggest having

two settling tanks.

8

Disinfectant

Tank

The

effluent

from

the

settling

tanks

is

collected in disinfection tank, where the

water is disinfected with chlorine injection.

A minimum contact time of 20 minutes is

recommended for disinfection.

9

Filter Feed

Tank

The

effluent

from

the

settling

tanks

is

collected in filter feed tank, where it is also

disinfected

by

injecting

Sodium

Hypochloride.

10

Pressure

Sand Filter

The effluent from the filter feed tanks is

further filtered in a pressure sand filter

with the help of a float controlled filter

feed pump. The filtration rate used in the

sand filter fill be 10 m3/m2

|  |  |  |  |  | transferred at a uniform rate to the treatment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | system. The anox tank is provided to nitrify |  |  |
|  |  |  |  |  | the nitrogen compounds present in the |  |  |
|  |  |  |  |  | wastewater. It is done in the absence of air |  |  |
|  |  |  |  | and with agitation for better mixing. | and with agitation for better mixing. |  |  |
|  |  |  | 6 Aerobic Effluent from the anaerobic treatment will |  |  |  |  |
|  |  |  | Treatment - further treat aerobically using MBBR | Treatment - further treat aerobically using MBBR |  |  |  |
|  |  |  | MBBR (Moving Bed Bio- Reactor) media. In this | MBBR (Moving Bed Bio- Reactor) media. In this |  |  |  |
|  |  |  |  |  | treatment system, the aeration tanks (2 |  |  |
|  |  |  |  |  | numbers) is filled with specially made |  |  |
|  |  |  |  |  | (imported) plastic body (called MBBR |  |  |
|  |  |  |  |  | Media) for the aggressive growth of useful |  |  |
|  |  |  |  |  | bactria in the tank for the reduction of |  |  |
|  |  |  |  |  | biological content. 50% fill of media is |  |  |
|  |  |  |  |  | recommened. The removal efficiency is |  |  |
|  |  |  |  | expected to be about 98%. |  |  |  |
|  |  |  | 7 Settling The effluent from MBBR tanks will |  |  |  |  |
|  |  |  | Tanks / contain dead cells of bacteria and high | Tanks / contain dead cells of bacteria and high |  |  |  |
|  |  |  | Clarifier content of suspended solids. To remove | Clarifier content of suspended solids. To remove |  |  |  |
|  |  |  |  |  | these matters, a secondary settling with |  |  |
|  |  |  |  |  | flocculation is suggested. In order to have |  |  |
|  |  |  |  |  | a better settling and if required a chemical |  |  |
|  |  |  |  |  | settling is provided. We suggest having |  |  |
|  |  |  | two settling tanks. |  |  |  |  |
|  |  |  | 8 Disinfectant The effluent from the settling tanks is |  |  |  |  |
|  |  |  | Tank collected in disinfection tank, where the | Tank collected in disinfection tank, where the |  |  |  |
|  |  |  |  |  | water is disinfected with chlorine injection. |  |  |
|  |  |  |  |  | A minimum contact time of 20 minutes is |  |  |
|  |  |  |  | recommended for disinfection. |  |  |  |
|  |  |  | 9 Filter Feed The effluent from the settling tanks is |  |  |  |  |
|  |  |  | Tank collected in filter feed tank, where it is also | Tank collected in filter feed tank, where it is also |  |  |  |
|  |  |  |  |  | disinfected by injecting Sodium |  |  |
|  |  |  | Hypochloride. |  |  |  |  |
|  |  |  | 10 Pressure The effluent from the filter feed tanks is |  |  |  |  |
|  |  |  | Sand Filter further filtered in a pressure sand filter | Sand Filter further filtered in a pressure sand filter |  |  |  |
|  |  |  |  |  | with the help of a float controlled filter |  |  |
|  |  |  |  |  | feed pump. The filtration rate used in the |  |  |
|  |  |  |  | sand filter fill be 10 m3/m2 |  |  |  |

11

Activate

Carbon

Filter

The effluent will further go through an

activated filter to remove colour odour and

residual chlorine.

12

Treated

Water

Storage

The final purified water is stored in the

treated water storage tank. This water is

recommended to reuse for flushing or

gardening etc. The excess water can be

drained to soak pit or drainage line.

13

Sludge Line

The sludge from the settling tank is further

directed to a sludge digestion tank. The

digested sludge from sludge tank can be

compacted in a filter press and disposed or

pumped out into a drying bed for further

disposal.

Capacity of Tanks:

Capacity of STP

30.0 KLD

Capacity of STP

4000.0 LPH

Liquid depth recommended

3.0 m

Oil and grease trap

1.2 m3

Equalization tank

48.0 m3

Anaerobic digestion tank for sewage

24.0 m3

Chemical settling tank for sullage

8.4 m3

Anox tank

8.0 m3

Capacity of MBBR Tank-1

10.0 m3

Capacity of MBBR Tank-2

10.0 m3

Clarifier-1

16.0 m3

Clarifier-2

Filter Feed tank

Treated water storage tank

16.0m3

8.0m3

8.0m3

UF Feed Tank

8.0 m3

|  |  |  | 11 Activate The effluent will further go through an |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 12 Treated The final purified water is stored in the |  |  |  |
|  |  |  |  | gardening etc. The excess water can be |  |  |
|  |  |  |  | drained to soak pit or drainage line. |  |  |
|  |  |  | 13 Sludge Line The sludge from the settling tank is further |  |  |  |
|  |  |  |  | directed to a sludge digestion tank. The |  |  |
|  |  |  |  | digested sludge from sludge tank can be |  |  |
|  |  |  |  | compacted in a filter press and disposed or |  |  |
|  |  |  |  | pumped out into a drying bed for further |  |  |
|  |  |  | disposal. |  |  |  |
|  | Capacity of Tanks: |  |  |  |  |  |
|  |  |  | Capacity of STP 30.0 KLD |  |  |  |
|  |  |  | Capacity of STP 4000.0 LPH |  |  |  |
|  |  |  | Liquid depth recommended 3.0 m |  |  |  |
|  |  |  | Oil and grease trap 1.2 m3 |  |  |  |
|  |  |  | Equalization tank 48.0 m3 |  |  |  |
|  |  |  | Anaerobic digestion tank for sewage 24.0 m3 |  |  |  |
|  |  |  | Chemical settling tank for sullage 8.4 m3 |  |  |  |
|  |  |  | Anox tank 8.0 m3 |  |  |  |
|  |  |  | Capacity of MBBR Tank-1 10.0 m3 |  |  |  |
|  |  |  | Capacity of MBBR Tank-2 10.0 m3 |  |  |  |
|  |  |  | Clarifier-1 16.0 m3 |  |  |  |
|  |  |  | Clarifier-2 16.0m3 |  |  |  |
|  |  |  | Filter Feed tank 8.0m3 |  |  |  |
|  |  |  | Treated water storage tank 8.0m3 |  |  |  |
|  |  |  | UF Feed Tank 8.0 m3 |  |  |  |

Slurry digester tank

8.0 m3

Total

100 m3

Power Requirement for STP:

HP

Hrs

KW/Day

KW/Month

1 Lift pump

2

20

30

900

2 Filter feed

Pump

2

10

15

450

3 Blowers

5

20

75

2,250

4 Ultra Filtration

3

20

45

1,350

5 Dosing pumps

0.1

20

2

45

Total

167

4,995

Expected Output Quality:

The treated water will meet the discharge standards suggested by KSPCB.

The expected levels are given below.

BOD

10 mg/l

COD

50 mg/l

pH

6.5 to 9

Suspended solids

10 mg/l

Amonia Nitrogen

5 mg/l

N- Total

10 mg/l

Fecal Coliform

<230 MPN/100 ml

|  |  |  | Slurry digester tank 8.0 m3 | Slurry digester tank 8.0 m3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Total 100 m3 | Total 100 m3 |  |  |  |  |
|  |  | Power Requirement for STP: |  |  |  |  |  |  |
|  |  |  |  |  |  | HP Hrs KW/Day KW/Month |  |  |
|  |  |  | 1 Lift pump 2 20 30 900 |  |  |  |  |  |
|  |  |  | 2 Filter feed 2 10 15 450 |  |  |  |  |  |
| Pump |  |  |  |  |  |  |  |  |
|  |  |  |  | 3 Blowers 5 20 75 2,250 |  |  |  |  |
|  |  |  |  | 4 Ultra Filtration 3 20 45 1,350 |  |  |  |  |
|  |  |  | 5 Dosing pumps 0.1 20 2 45 |  |  |  |  |  |
|  |  |  |  | Total 167 4,995 |  |  |  |  |
|  |  | Expected Output Quality: |  |  |  |  |  |  |
|  |  |  |  | The treated water will meet the discharge standards suggested by KSPCB. |  |  |  |  |
|  |  |  | pH 6.5 to 9 |  |  |  |  |  |
|  |  |  | Fecal Coliform <230 MPN/100 ml |  |  |  |  |  |

Sl. No.

ITEM

SPECIFICATION

UNIT

QTY

1

Lift pump

1 on duty

nos.

2

Make/ Brand

Kirloskar

Type/ Model

Self- priming sewage

pump

Material of Construction

CI

HP/KW ratings

2 HP

Pipe size

2"

Capacity

5 m2/hour at 12 m

2

Filter feed Pump

1 on duty

nos.

2

Make/ Brand

Kirloskar/ Johnson/

Wilo

Type/ Model

Cetrifugal

Material of Construction

CI

HP/KW ratings

2 HP, 2 Ph

Pipe size

1.5"

Capacity

5 m2/hour at 25 m

3

Blowers

1 on duty

nos.

2

Make/ Brand

A1/Akash

Type/ Model

Twin lobe blowers

Material of Construction

CI

Motor Make

Siemens/ Kirloskar

HP/KW ratings

5HP

Pipe size

1.5"

Capacity

200 m2/hour at 3 m

water

|  |  |  | Sl. No. ITEM SPECIFICATION UNIT QTY |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 Lift pump 1 on duty nos. 2 |  |  |  |
|  |  |  | Type/ Model Self- priming sewage |  |  |  |
|  |  |  | pump |  |  |  |
|  |  | Material of Construction CI |  |  |  |  |
|  |  | HP/KW ratings 2 HP |  |  |  |  |
|  |  | Pipe size 2" |  |  |  |  |
|  |  |  | 2 Filter feed Pump 1 on duty nos. 2 |  |  |  |
|  |  |  | Wilo |  |  |  |
|  |  | Material of Construction CI |  |  |  |  |
|  |  | Pipe size 1.5" |  |  |  |  |
|  |  |  | 3 Blowers 1 on duty nos. 2 |  |  |  |
|  |  | Material of Construction CI |  |  |  |  |
|  |  | HP/KW ratings 5HP |  |  |  |  |
|  |  | Pipe size 1.5" |  |  |  |  |
|  |  |  | water |  |  |  |

4

MBBR media

Material of Construction

Type/ Model

Make:

Imported Bio-wheels

on

PP

K3

type

Vasuenviro products

m3

8

5

Diffusers

nos.

16

Type/ Model

Make

Dimensions

fine bubble tubular

Anjeneya

International

65x 650 mm

6

Dosing pump

for Chlorine dosing

nos.

3

Make/ Brand

Pentair/edose

Type/ Model

Diaphram pumps

Material of Construction

Capacity

Liquid contact is with

PP

0-5 LPH

7

Pressure Sand Filter

nos.

1

Dimensions

Material of Construction

dia 760 &Ht>1600 mm

FRP

Make:

Pentair/ Aventura

Media

Graded Quartz sand

8

Activated carbon Filter

nos.

1

Dimensions

Material of Construction

dia 760 &Ht>1600 mm

FRP

Make:

Pentair/ Aventura

Media

Activated carbon IV-

1000

9

Control Panel and electrical works

nos.

1

|  |  |  |  | 4 MBBR media m3 8 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 Diffusers nos. 16 |  |  |  |  |
|  |  |  | Make Anjeneya |  |  |  |  |  |
|  |  |  | Dimensions International |  |  |  |  |  |
|  |  |  |  |  | 65x 650 mm |  |  |  |
|  |  |  |  | 6 Dosing pump for Chlorine dosing nos. 3 |  |  |  |  |
|  |  |  | Make/ Brand Pentair/edose |  |  |  |  |  |
|  |  |  | Type/ Model Diaphram pumps |  |  |  |  |  |
|  |  |  |  | Material of Construction Liquid contact is with |  |  |  |  |
|  |  | Capacity PP | Capacity PP |  |  |  |  |  |
|  |  |  |  | 0-5 LPH |  |  |  |  |
|  |  |  |  | 7 Pressure Sand Filter nos. 1 |  |  |  |  |
|  |  |  |  | Dimensions dia 760 &Ht>1600 mm |  |  |  |  |
|  |  |  | Material of Construction FRP |  |  |  |  |  |
|  |  |  | Make: Pentair/ Aventura |  |  |  |  |  |
|  |  |  |  | 8 Activated carbon Filter nos. 1 |  |  |  |  |
|  |  |  |  | Dimensions dia 760 &Ht>1600 mm |  |  |  |  |
|  |  |  | Material of Construction FRP |  |  |  |  |  |
|  |  |  | Make: Pentair/ Aventura |  |  |  |  |  |
|  |  |  |  | 1000 |  |  |  |  |
|  |  |  |  | 9 Control Panel and electrical works nos. 1 |  |  |  |  |

Make/ Brand

L&T Fittings

Fittings

Starter, timer controlled

operation

changeover

switch for pumps and

TOD type energy meter

Material of Construction

MS

powder

coated

CRCA 16 guage sheet

1

0

Water meter 1.5 " inlet/outlet

nos.

1

1

1

Pipe and fittings

Lot

(includes

all

plumping and electrical

material)

Lot

1

Make/Brand

Type/Model

Supreme/ Fenolex

uPVC pipes

1

2

Instllation and commissioning

Lot

1

1

3

Ultra filtration

Capacity:

4 m3/hour

Set

1

No. of membranes

No of skid

No of streams

Feed pump

2

1 (fabricated on

MS) 1

1 (SS - Centrifugal) 1 .5

HP

Backwash Pump

1 (Centrifugal) 2 HP

Membrane:

Hitec

Control panel

PLC based automatic

control

|  |  |  | Fittings Starter, timer controlled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Material of Construction MS powder coated |  |  |  |
|  |  |  |  | CRCA 16 guage sheet |  |  |
|  |  |  | 1 Water meter 1.5 " inlet/outlet nos. 1 |  |  |  |
| 0 |  |  |  |  |  |  |
|  |  |  | 1 Pipe and fittings Lot (includes all Lot 1 |  |  |  |
|  |  |  | material) | material) |  |  |
|  |  |  | 1 Instllation and commissioning Lot 1 |  |  |  |
| 2 |  |  |  |  |  |  |
|  |  |  | 1 Ultra filtration Set 1 |  |  |  |
|  |  | 3 Capacity: 4 m3/hour |  |  |  |  |
|  |  | No. of membranes 2 |  |  |  |  |
|  |  |  | Feed pump 1 (SS - Centrifugal) 1 .5 |  |  |  |
|  |  |  | HP |  |  |  |
|  |  |  | Backwash Pump 1 (Centrifugal) 2 HP |  |  |  |
|  |  | Membrane: Hitec |  |  |  |  |
|  |  |  | Control panel PLC based automatic |  |  |  |
|  |  |  | control |  |  |  |

CHAPTER 3: SOLID WASTE MANAGEMENT

3.1

1: Construction Phase:

The total man power

=

100

Considering solid waste generation @ 0.20 kg/capita/day Total

solid waste generation = 100 x 0.20 = 20 kg/day

The domestic waste will be disposed through corporation authorities.

3.2

2: Occupancy Phase:

The wastes that are generated from day to day activities which are in

solid form are categorized as solid wastes. Solid waste includes dry refuse of

offices, street sweepings, and also electronic wastes. The quantity of solid waste

generated from the proposed project is calculated as follows:

Details of solid waste generation:

Sl.No.

Description

Population

Solid waste generated kg/day

Total

Organic

(40%)

Inorganic

(60%)

1

Building

500

500 x 0.20

=100

40

60

Total

100

kg/day

Considering solid waste generated in office as 0.20 kg/day and 40% of the solid

waste generated is organic waste.

The solid wastes generated will be segregated at its point generation and collected

separately in different colour coded synthetic Bins depending upon the basis of

its Biodegradability at a common designated point. The collection and disposal

of

the

solid

wastes

from

the

project

is

done

in

such

a

way

that

it

is

scientifically disposed without causing eyesore or any adverse effects to the

surrounding environment. Organic sold waste from the site will be treated in an

|  |  |  |  |  | CHAPTER 3: SOLID WASTE MANAGEMENT |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3.1 1: Construction Phase: |  |  |  |  |  |  |  |
|  |  |  | The total man power = 100 |  |  |  |  |  |  |
|  |  |  |  |  | Considering solid waste generation @ 0.20 kg/capita/day Total |  |  |  |  |
|  |  |  |  | solid waste generation = 100 x 0.20 = 20 kg/day |  |  |  |  |  |
|  |  |  |  |  | The domestic waste will be disposed through corporation authorities. |  |  |  |  |
|  |  | 3.2 2: Occupancy Phase: |  |  |  |  |  |  |  |
|  |  |  |  |  | solid form are categorized as solid wastes. Solid waste includes dry refuse of |  |  |  |  |
|  |  |  |  |  | offices, street sweepings, and also electronic wastes. The quantity of solid waste |  |  |  |  |
|  |  |  |  | generated from the proposed project is calculated as follows: |  |  |  |  |  |
|  |  | Details of solid waste generation: |  |  |  |  |  |  |  |
|  |  |  |  |  | Sl.No. Description Population Solid waste generated kg/day |  |  |  |  |
|  |  |  |  |  |  |  |  | Total Organic Inorganic |  |
|  |  |  |  |  |  |  |  | (40%) (60%) |  |
|  |  |  |  |  | 1 Building 500 500 x 0.20 40 60 |  |  |  |  |
|  |  |  |  |  |  | =100 |  |  |  |
|  |  |  |  |  |  |  | Total 100 |  |  |
|  |  |  |  |  |  |  |  | kg/day |  |
|  |  |  |  |  | Considering solid waste generated in office as 0.20 kg/day and 40% of the solid |  |  |  |  |
|  |  | waste generated is organic waste. |  |  |  |  |  |  |  |
|  |  |  |  |  | The solid wastes generated will be segregated at its point generation and collected |  |  |  |  |
|  |  |  |  |  | separately in different colour coded synthetic Bins depending upon the basis of |  |  |  |  |
|  |  |  |  |  | its Biodegradability at a common designated point. The collection and disposal |  |  |  |  |
|  |  |  |  |  | of the solid wastes from the project is done in such a way that it is |  |  |  |  |
|  |  |  |  |  | scientifically disposed without causing eyesore or any adverse effects to the |  |  |  |  |
|  |  |  |  |  | surrounding environment. Organic sold waste from the site will be treated in an |  |  |  |  |

organic waste converter and is used as manure for landscape or used as fuel.

The inorganic solid waste is proposed to be recycled.

Operational Procedure:

Collection of organic

wastes in a container

Feed the waste in the machine after

draining excess water

Mix culture and absorbent with

waste

Crush and mix (10 min)

Collect crushed pulp in a tray

for aeration and drying

Repeat the feeding batches till

material finishes

Recycle dry pulp with next fresh waste in (10:4)

and remaining as final product.

Clean the machine at the end of operation.

|  |  |  | organic waste converter and is used as manure for landscape or used as fuel. |  |
| --- | --- | --- | --- | --- |
|  |  | The inorganic solid waste is proposed to be recycled. |  |  |
| Operational Procedure: |  |  |  |  |
| Collection of organic |  |  |  |  |
| wastes in a container |  |  |  |  |
|  |  |  |  | Feed the waste in the machine after |
|  |  |  |  | draining excess water |
|  |  |  |  | Mix culture and absorbent with |
|  |  |  |  | waste |
|  |  |  |  | Crush and mix (10 min) |
|  | Collect crushed pulp in a tray |  |  |  |
|  | for aeration and drying |  |  |  |
|  |  |  |  | Repeat the feeding batches till |
|  |  |  |  | material finishes |
|  |  | Recycle dry pulp with next fresh waste in (10:4) |  |  |
|  |  | and remaining as final product. |  |  |
|  |  | Clean the machine at the end of operation. |  |  |

Process Flow Chart for Organic Waste Converter:

Organic waste

Waste treatment 10 min

Output (Homogenized and odour free)

Sundrying for 2 days

Curing 15- 20 days in a

container

Water/room heating

application

Landscaping application

3.2.1 Secondary Sludge from STP:

The solid waste generated from the STP of secondary settling is in the

form of stabilized secondary sludge. Then it is passed through the plate and

frame fitter press. The solids obtained as semi solid cakes which can be used as

organic manure for the development of landscaping.

Rate:

80M3/day and sludge: 12 kgs/day (considering 15% of sewage volume as

sludge generation)

|  |  |  | Process Flow Chart for Organic Waste Converter: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Organic waste |  |  |
|  |  |  |  | Waste treatment 10 min |  |  |
|  |  |  |  | Output (Homogenized and odour free) |  |  |
|  |  |  |  | Sundrying for 2 days Curing 15- 20 days in a |  |  |
|  |  |  |  |  | container |  |
|  | Water/room heating |  |  |  |  |  |
|  |  |  |  | application Landscaping application |  |  |
|  |  | 3.2.1 Secondary Sludge from STP: |  |  |  |  |
|  |  |  |  | The solid waste generated from the STP of secondary settling is in the |  |  |
|  |  |  |  | form of stabilized secondary sludge. Then it is passed through the plate and |  |  |
|  |  |  |  | frame fitter press. The solids obtained as semi solid cakes which can be used as |  |  |
|  |  |  | organic manure for the development of landscaping. |  |  |  |
|  |  |  |  | Rate: 80M3/day and sludge: 12 kgs/day (considering 15% of sewage volume as |  |  |
|  | sludge generation) |  |  |  |  |  |

3.2.2Hazardous Waste:

The hazardous Waste generated from the project is waste oil from DG

sets which will be stored in HDPE barrels and disposed.

3.2.3 Bio Medical Waste:

No

bio medical waste is generated on site, incase of bio medical waste

generated from the project will be collected and segregated at source based on

their categorization as per the Bio medical waste rule in containers and will be

disposed as per guidelines. Common designated place will be earmarked for the

storage of biomedical waste and will be disposed scientifically.

3.2.4

: e-Waste:

The

electronic

waste

such

as

CD’s

pen

drives,

computer

and

its

components, used batteries etc. from the project will be segregated collected and

stored at a designated place and will be handed over to authorized recyclers.

3.3

3: Disposal of Excavated Earth and Construction Debris:

3.3.1 Excavated Earth Disposal:

The

proposed building in the

project

has

basement

floor

and

earth

excavation is necessary. Quantity of earth excavated and its disposal is as given

below.

Sl.

No.

Description

Basement Area (M

2

)

Basement

height (M)

earth

excavated

(M

3

)

1

Building

1900

4.2

7980

The excavated earth will be reused for back filling, paved area formation activity

and landscape development. The excess excavated earth will be disposed off at

pre-identified site.

For back filling: Considering 35% of the total excavated earth to be used

= 7980x 35%

= 3990 cu.m

For landscaping: Considering 50cm filling

= 200 sq.m x 0.50

= 100cu.m

|  |  | 3.2.2Hazardous Waste: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | The hazardous Waste generated from the project is waste oil from DG |  |  |  |
|  |  |  | sets which will be stored in HDPE barrels and disposed. |  |  |  |  |  |  |
|  |  | 3.2.3 Bio Medical Waste: |  |  |  |  |  |  |  |
|  |  |  |  |  |  | No bio medical waste is generated on site, incase of bio medical waste |  |  |  |
|  |  |  |  |  | generated from the project will be collected and segregated at source based on |  |  |  |  |
|  |  |  |  |  | their categorization as per the Bio medical waste rule in containers and will be |  |  |  |  |
|  |  |  |  |  | disposed as per guidelines. Common designated place will be earmarked for the |  |  |  |  |
|  |  |  |  | storage of biomedical waste and will be disposed scientifically. |  |  |  |  |  |
|  | 3.2.4 : e-Waste: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | The electronic waste such as CD’s pen drives, computer and its |  |  |  |
|  |  |  |  |  | components, used batteries etc. from the project will be segregated collected and |  |  |  |  |
|  |  |  |  |  | stored at a designated place and will be handed over to authorized recyclers. |  |  |  |  |
|  |  |  |  |  | 3.3 3: Disposal of Excavated Earth and Construction Debris: |  |  |  |  |
|  |  | 3.3.1 Excavated Earth Disposal: |  |  |  |  |  |  |  |
|  |  |  |  |  |  | The proposed building in the project has basement floor and earth |  |  |  |
|  |  |  |  |  | excavation is necessary. Quantity of earth excavated and its disposal is as given |  |  |  |  |
| below. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Sl. Description Basement Area (M ) Basement earth | 2 |  |  |  |
|  |  |  |  |  | No. height (M) excavated |  |  |  |  |
|  |  |  |  |  |  |  |  | (M ) |  |
|  |  |  |  |  | 1 Building 1900 4.2 7980 |  |  |  |  |
|  |  |  |  |  | The excavated earth will be reused for back filling, paved area formation activity |  |  |  |  |
|  |  |  |  |  | and landscape development. The excess excavated earth will be disposed off at |  |  |  |  |
|  | pre-identified site. |  |  |  |  |  |  |  |  |
|  |  |  |  |  | For back filling: Considering 35% of the total excavated earth to be used |  |  |  |  |
|  |  |  | = 7980x 35% |  |  |  |  |  |  |
|  |  |  | = 3990 cu.m |  |  |  |  |  |  |
|  |  |  | For landscaping: Considering 50cm filling |  |  |  |  |  |  |
|  |  |  | = 200 sq.m x 0.50 | = 200 sq.m x 0.50 |  |  |  |  |  |
|  |  |  | = 100cu.m |  |  |  |  |  |  |

For paved area development: Considering 50 cm filling:

= 1000.00 Sq.m x 0.50

=500M

3

Total quantity of excavated earth to be reused = 4600 Cu.m

3.3.2

: Construction Debris:

Construction debris generated will be used as preparatory for formation

activities within the project site. Necessary measures and modern construction

practices such as proportioning and weigh batchers for concreting etc. will be

implemented to reduce construction debris.

|  |  | For paved area development: Considering 50 cm filling: |  |
| --- | --- | --- | --- |
|  |  | = 1000.00 Sq.m x 0.50 |  |
|  |  | 3 |  |
|  |  | Total quantity of excavated earth to be reused = 4600 Cu.m |  |
|  | 3.3.2 : Construction Debris: |  |  |
|  |  |  | Construction debris generated will be used as preparatory for formation |
|  |  |  | activities within the project site. Necessary measures and modern construction |
|  |  |  | practices such as proportioning and weigh batchers for concreting etc. will be |

CHAPTER 4: AIR AND NOISE POLLUTION

MANAGEMENT

4.1

1: Air Pollution Source and its Management:

The anticipated power required for the project (About 1000 KW) will be

supplied from KSEB. The primary sources of air pollution from the establishment

are from the operation of diesel generator set. The project will be provided with

DG set of 2 nos of 500KVAcapacity and it is installed to serve as an alternative

back up source of power supply in the event of breakdown of power supply

from KSEB. The details of capacities and fuel consumption for the DG sets are

given below.

Sl. No.

Stack details

Stack attached to

Physical details

D.G. set.

1

Capacity

2 x 500 KVA

2

Fuel quantity

108 Liters/hr

3

Fuel used

High speed diesel

4

Stack height

Chimney above roof level

5

Stack diameter

60 cm

Emission Details:

1

Sulpher Dioxide

0.097g/s

2

Suspended

particulate

matter (SPM)

0.03 g/s

3

Oxides of nitrogen

0.57g/s

4.2

2: Noise Generation Sources:

Major noise producing sources of the commercial project is expected to

be from DG sets and vehicular movements from and to the site. The DG sets

are provided with acoustic enclosures to control the noise levels in such a way

that the noise levels are within the permissible limits specified for ambient noise

levels. Moreover DG sets are operated only during emergencies when there is

power failure.

|  |  |  |  |  | CHAPTER 4: AIR AND NOISE POLLUTION |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MANAGEMENT |  |  |
|  |  |  |  |  | The anticipated power required for the project (About 1000 KW) will be |  |  |
|  |  |  |  |  | supplied from KSEB. The primary sources of air pollution from the establishment |  |  |
|  |  |  |  |  | are from the operation of diesel generator set. The project will be provided with |  |  |
|  |  |  |  |  | DG set of 2 nos of 500KVAcapacity and it is installed to serve as an alternative |  |  |
|  |  |  |  |  | back up source of power supply in the event of breakdown of power supply |  |  |
|  |  |  |  |  | from KSEB. The details of capacities and fuel consumption for the DG sets are |  |  |
|  | given below. |  |  |  |  |  |  |
|  |  |  |  | Sl. No. Stack details Stack attached to |  |  |  |
|  |  |  |  | Physical details D.G. set. |  |  |  |
|  |  |  | 1 Capacity 2 x 500 KVA |  |  |  |  |
|  |  |  | 2 Fuel quantity 108 Liters/hr |  |  |  |  |
|  |  |  | 3 Fuel used High speed diesel |  |  |  |  |
|  |  |  |  | 4 Stack height Chimney above roof level |  |  |  |
|  |  |  | 5 Stack diameter 60 cm |  |  |  |  |
|  | Emission Details: |  |  |  |  |  |  |
|  |  |  | 1 Sulpher Dioxide 0.097g/s |  |  |  |  |
|  |  |  | 2 Suspended particulate 0.03 g/s |  |  |  |  |
|  |  | matter (SPM) |  |  |  |  |  |
|  |  |  | 3 Oxides of nitrogen 0.57g/s |  |  |  |  |
|  |  | 4.2 2: Noise Generation Sources: |  |  |  |  |  |
|  |  |  |  |  | Major noise producing sources of the commercial project is expected to |  |  |
|  |  |  |  |  | be from DG sets and vehicular movements from and to the site. The DG sets |  |  |
|  |  |  |  |  | are provided with acoustic enclosures to control the noise levels in such a way |  |  |
|  |  |  |  |  | that the noise levels are within the permissible limits specified for ambient noise |  |  |
|  |  |  |  |  | levels. Moreover DG sets are operated only during emergencies when there is |  |  |
|  | power failure. |  |  |  |  |  |  |

CHAPTER 5: RAIN WATER HARVESTING AND

STORM WATER MANAGEMENT

5.1

1: Rain Water Harvesting and Storm Water Management:

The water collected from the terrace of the building shall be collected in

the rain water collection sump and reused for domestic purpose after necessary

treatment.

5.2

2: Volume of Rain Water Harvested:

The total quantity of water i.e., received in the form of rail fall over an

area is called the rain water endowment of that area out of which the amount of

water than can be effectively harvested is called the rainwater harvesting

potential.

Rain water harvesting potential =

Intensity of rain fall (m) x

Roof area x Impermeability factor.

The collection efficiency accounts for the fact that all the rainwater falling

over an area cannot be effectively harvested due to losses on account of

evaporation, spillage or run off etc.

According to data available from the Indian Meteorological Department,

the average annual rainfall is 1423.90 as per IMD. Assuming that about 90%

rain fall can be effectively harvested.

Number of raining days

=

69

Therefore the IR = 1423.90/60 = 20.64 mm/day or 0.021 m/day Rain

water Q = 0.021 x 75% roof area x 0.90

Sl.

No.

Description

Roof Area/sq.m

75% of Roof

area (Sq.m)

Quantity of

rainwater

harvested

M3

1

Building

578

433.5

8.19

Total quantity of rainwater harvested = say 8.19 cu.m

5.3

3: Storm Management:

Storm water disposal is divided into 2 Groups:

|  |  |  |  | CHAPTER 5: RAIN WATER HARVESTING AND |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | STORM WATER MANAGEMENT |  |  |
|  |  |  |  | 5.1 1: Rain Water Harvesting and Storm Water Management: |  |  |
|  |  |  |  | the rain water collection sump and reused for domestic purpose after necessary |  |  |
|  |  | 5.2 2: Volume of Rain Water Harvested: |  |  |  |  |
|  |  |  |  | area is called the rain water endowment of that area out of which the amount of |  |  |
|  |  |  |  | water than can be effectively harvested is called the rainwater harvesting |  |  |
|  |  |  | Rain water harvesting potential = Intensity of rain fall (m) x |  |  |  |
|  |  |  |  | over an area cannot be effectively harvested due to losses on account of |  |  |
|  |  |  |  | the average annual rainfall is 1423.90 as per IMD. Assuming that about 90% |  |  |
|  |  | Number of raining days = 69 |  |  |  |  |
|  |  |  |  | Therefore the IR = 1423.90/60 = 20.64 mm/day or 0.021 m/day Rain |  |  |
|  |  | water Q = 0.021 x 75% roof area x 0.90 |  |  |  |  |
|  |  |  |  | Sl. Description Roof Area/sq.m 75% of Roof Quantity of |  |  |
|  |  |  |  | No. area (Sq.m) rainwater |  |  |
|  |  |  |  |  |  | M3 |
|  |  |  | 1 Building 578 433.5 8.19 |  |  |  |
|  |  | Storm water disposal is divided into 2 Groups: |  |  |  |  |

a)

Terrace storm water disposal

: The entire rainwater from the terrace

would be disposed through suitable rainwater pipes and collecting in the

dedicated rain water

collection

sump. This water will

be

utilized for

domestic purpose.

b)

Site storm water disposal

: The entire storm water from the site would

be

disposed

off

through

suitable

RCC

box

drainage

system to

the

rainwater recharge pits and the excess is diverted to external storm water

drainage.

The amount of storm water that the paved area will produce can be

determined by considering the impermeability factor to be 0.90. The

paved area of the proposed project is 1275.00 M

2

Q

= 0.021 x

1275.00 x 0.9

=

24 cum/day

The proponent shall also provide recharge pits along the boundary with

recharge pits of size 1.20 m dia x 2.00 m deep spaced at 20m centre to

centre. The pits are filled with graded media comprising of boulder at

bottom and coarse aggregate.

|  |  |  |  | a) Terrace storm water disposal : The entire rainwater from the terrace |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | would be disposed through suitable rainwater pipes and collecting in the |  |  |  |
|  |  |  |  |  | dedicated rain water collection sump. This water will be utilized for |  |  |  |
|  |  | domestic purpose. |  |  |  |  |  |  |
|  |  |  |  | b) Site storm water disposal : The entire storm water from the site would |  |  |  |  |
|  |  |  |  |  | be disposed off through suitable RCC box drainage system to the |  |  |  |
|  |  |  |  |  | rainwater recharge pits and the excess is diverted to external storm water |  |  |  |
|  | drainage. |  |  |  |  |  |  |  |
|  |  |  |  |  | The amount of storm water that the paved area will produce can be |  |  |  |
|  |  |  |  |  | determined by considering the impermeability factor to be 0.90. The |  |  |  |
|  |  |  | paved area of the proposed project is 1275.00 M |  |  | 2 |  |  |
|  |  |  | Q = 0.021 x 1275.00 x 0.9 = 24 cum/day |  |  |  |  |  |
|  |  |  |  |  | The proponent shall also provide recharge pits along the boundary with |  |  |  |
|  |  |  |  |  | recharge pits of size 1.20 m dia x 2.00 m deep spaced at 20m centre to |  |  |  |
|  |  |  |  |  | centre. The pits are filled with graded media comprising of boulder at |  |  |  |
|  |  | bottom and coarse aggregate. |  |  |  |  |  |  |

CHAPTER 6: ENVIRONMENTAL

MANAGEMENT PLAN

6.1

1: Environmental Management Plan:

The environmental management plan (EMP) is aimed at mitigating the

possible adverse impact of a project and ensure the existing environmental

quality. The EMP converse all aspects of planning, construction and operation

of the project relevant to Environment. It is essential to implement the EMP

right

from the

planning

stage

continuing

throughout

the

construction

and

operation stage. Therefore the main purpose of the environmental management

plan is to identify the project specific activities that would have to be considered

for the significant adverse impacts and the mitigation measures required. The

construction phase impacts are mostly short term restricted to the plot area and

not envisaged on the large scale. In the operational phase the environmental

impacts are due to continuous operation of the project. Hence the emphasis in

the environment management plan is to minimize such impacts. The following

mitigation measures are recommended in order to synchronize the economic

development of the project area with the environmental protection of the region.

The emphasis on the EMP development is on the following

*

Mitigation measures for each of the activities causing the environment impact

*

Monitoring plans for checking activities and environmental parameters

*

Role responsibilities and resource allocation for monitoring

*

Implementation of the scheduled plan.

6.2

2: EMP During Construction Phase:

During construction phase activities to be monitored are given below.

Environmental Impact

Mitigation Proposed

Remarks

Noise generation caused

due

to

demolition

of

machineries,

excavators

and Bulldozers.

*Most

optimum

no.

of

operation

by

the

heavy

equipment.

*Selection

of

equipment

with less noise generation.

*The

earth

moving

equipment

shall

be

periodically checked.

To reduce noise level

equipment

provided

with

noise

control

devises is only used.

|  |  |  |  |  | CHAPTER 6: ENVIRONMENTAL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MANAGEMENT PLAN |  |  |  |  |  |  |
|  |  |  | 6.1 1: Environmental Management Plan: |  |  |  |  |  |  |  |
|  |  |  |  |  |  | The environmental management plan (EMP) is aimed at mitigating the |  |  |  |  |
|  |  |  |  |  | possible adverse impact of a project and ensure the existing environmental |  |  |  |  |  |
|  |  |  |  |  | quality. The EMP converse all aspects of planning, construction and operation |  |  |  |  |  |
|  |  |  |  |  | of the project relevant to Environment. It is essential to implement the EMP |  |  |  |  |  |
|  |  |  |  |  | right from the planning stage continuing throughout the construction and |  |  |  |  |  |
|  |  |  |  |  | operation stage. Therefore the main purpose of the environmental management |  |  |  |  |  |
|  |  |  |  |  | plan is to identify the project specific activities that would have to be considered |  |  |  |  |  |
|  |  |  |  |  | for the significant adverse impacts and the mitigation measures required. The |  |  |  |  |  |
|  |  |  |  |  | construction phase impacts are mostly short term restricted to the plot area and |  |  |  |  |  |
|  |  |  |  |  | not envisaged on the large scale. In the operational phase the environmental |  |  |  |  |  |
|  |  |  |  |  | impacts are due to continuous operation of the project. Hence the emphasis in |  |  |  |  |  |
|  |  |  |  |  | the environment management plan is to minimize such impacts. The following |  |  |  |  |  |
|  |  |  |  |  | mitigation measures are recommended in order to synchronize the economic |  |  |  |  |  |
|  |  |  |  |  | development of the project area with the environmental protection of the region. |  |  |  |  |  |
|  |  |  |  | The emphasis on the EMP development is on the following |  |  |  |  |  |  |
|  |  |  |  |  | * Mitigation measures for each of the activities causing the environment impact |  |  |  |  |  |
|  |  |  |  |  | * Monitoring plans for checking activities and environmental parameters |  |  |  |  |  |
|  |  |  |  | * Role responsibilities and resource allocation for monitoring |  |  |  |  |  |  |
|  |  |  | 6.2 2: EMP During Construction Phase: |  |  |  |  |  |  |  |
|  |  |  |  | During construction phase activities to be monitored are given below. | During construction phase activities to be monitored are given below. |  |  |  |  |  |
|  |  |  |  |  | Environmental Impact Mitigation Proposed Remarks |  |  |  |  |  |
|  |  |  |  |  | Noise generation caused *Most optimum no. of To reduce noise level |  |  |  |  |  |
|  |  |  |  |  | due to demolition of operation by the heavy equipment provided |  |  |  |  |  |
|  |  |  |  |  | machineries, excavators equipment. with noise control |  |  |  |  |  |
|  |  |  |  |  | and Bulldozers. *Selection of equipment devises is only used. |  |  |  |  |  |
|  |  |  |  |  | with less noise generation. |  |  |  |  |  |
|  |  |  |  |  | *The earth moving |  |  |  |  |  |
|  |  |  |  |  | equipment shall be |  |  |  |  |  |
|  |  |  |  |  | periodically checked. |  |  |  |  |  |

*The

workers

will

be

provided with PPE

Environmental

Impacts

Mitigation Proposed

Remarks

Dust

generation:

leveling

and

demolition activities

results

in

the

emission of dust.

The

site

cleared

will

be

periodically

watered

to

reduce

dust emissions.

Bar codes like metal sheets will be

provided all around the premises

to

avoid

fugitive

dust

emission

during

demolition

to

the

neighboring area apart from water

sprinkling

Workers

will

be

provided

with

PPE

such

as

nose

masks

and

goggles to reduce impact.

Treated water to be

used.

Transportation of construction materials:

Environmental Impacts

Mitigation Proposed

Noise generation

*Quality fuel will be used

*Periodic maintenance of vehicle is required.

Dust generation

*Quality

packaging

of

the

construction

materials

*Construction materials will be covered with

tarpaulin sheet to prevent from being airborne.

*The vehicle speed shall be regulated.

*The workers, transporting materials will be

provided with PPE kits such as nose masks.

Vehicular emission

*Periodic emission check for vehicles will be

done.

*Clean fuel will be used for vehicles.

|  |  |  |  |  | *The workers will be |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | provided with PPE | provided with PPE |  |  |  |  |
|  |  |  |  |  | Environmental Mitigation Proposed Remarks |  |  |  |  |
|  | Impacts |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Dust generation: The site cleared will be Treated water to be |  |  |  |  |
|  |  |  |  | leveling and periodically watered to reduce used. |  |  |  |  |  |
|  |  | demolition activities dust emissions. |  |  |  |  |  |  |  |
|  | results in the |  |  |  |  |  |  |  |  |
|  |  |  | emission of dust. Bar codes like metal sheets will be |  |  |  |  |  |  |
|  |  |  |  |  | provided all around the premises |  |  |  |  |
|  |  |  |  |  | to avoid fugitive dust emission |  |  |  |  |
|  |  |  |  |  | during demolition to the |  |  |  |  |
|  |  |  |  |  | neighboring area apart from water |  |  |  |  |
|  |  |  | sprinkling |  |  |  |  |  |  |
|  |  |  |  |  | Workers will be provided with |  |  |  |  |
|  |  |  |  |  | PPE such as nose masks and |  |  |  |  |
|  |  |  |  | goggles to reduce impact. |  |  |  |  |  |
|  |  | Transportation of construction materials: | Transportation of construction materials: |  |  |  |  |  |  |
|  |  |  |  | Environmental Impacts Mitigation Proposed |  |  |  |  |  |
|  |  |  |  |  | Noise generation *Periodic maintenance of vehicle is required. |  |  |  |  |
|  |  |  |  |  | *Quality fuel will be used | *Quality fuel will be used |  |  |  |
|  |  | Dust generation materials |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | *Quality packaging of the construction |  |  |
|  |  |  |  |  |  |  | *Construction materials will be covered with |  |  |
|  |  |  |  |  |  |  | tarpaulin sheet to prevent from being airborne. |  |  |
|  |  |  |  |  |  | *The vehicle speed shall be regulated. | *The vehicle speed shall be regulated. |  |  |
|  |  |  |  |  |  |  | *The workers, transporting materials will be |  |  |
|  |  |  |  |  |  |  | provided with PPE kits such as nose masks. |  |  |
|  |  | Vehicular emission done. |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | *Periodic emission check for vehicles will be |  |  |
|  |  |  |  |  |  | *Clean fuel will be used for vehicles. | *Clean fuel will be used for vehicles. |  |  |

Construction Activities:

Environmental

Impacts

Mitigation Proposed

Remarks

Noise generation

*Less

noise

generation

equipment

*PPE such as ear plugs and

helmets will be provided for

workers

*Working

hours

to

be

imposed on the construction

workers.

Implementation

responsibility:

Contractor

–

Civil Works

Dust generation

*PPE in the form of noise

masks will be provided for

construction workers.

*Use

of

water

sprays

to

prevent dust from being air

borne

*Barricade like metal sheets

will be provided all around

the

premises

to

avoid

fugitive dust emission into

the neighboring area.

Implementation

responsibility

–

Contractor

Dust generation

*PPE in the form of noise

masks will be provided for

construction workers.

*Use

of

water

sprays

to

prevent dust from being air

borne

*Barricade like metal sheets

will be provided all around

the

premises

to

avoid

fugitive dust emission into

the neighbouring area.

Dust generation

*PPE in the form of noise

Implementation

|  |  |  | Environmental Mitigation Proposed Remarks |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Impacts |  |  |  |  |  |  |  |
|  |  |  | Noise generation *Less noise generation Implementation |  |  |  |  |
|  |  |  |  | equipment responsibility: |  |  |  |
|  |  |  |  | *PPE such as ear plugs and Civil Works |  |  |  |
|  |  |  | helmets will be provided for |  |  |  |  |
|  | workers |  |  |  |  |  |  |
|  |  |  | *Working hours to be |  |  |  |  |
|  |  |  | imposed on the construction |  |  |  |  |
|  | workers. | workers. |  |  |  |  |  |
|  |  |  |  |  | Contractor – |  |  |
|  |  |  | Dust generation *PPE in the form of noise Implementation |  |  |  |  |
|  |  |  |  | construction workers. Contractor |  |  |  |
|  |  |  | *Use of water sprays to |  |  |  |  |
|  |  |  | prevent dust from being air |  |  |  |  |
|  | borne |  |  |  |  |  |  |
|  |  |  | *Barricade like metal sheets |  |  |  |  |
|  |  |  | will be provided all around |  |  |  |  |
|  |  |  | the premises to avoid |  |  |  |  |
|  |  |  | fugitive dust emission into |  |  |  |  |
|  |  | the neighboring area. | the neighboring area. |  |  |  |  |
|  | Dust generation *PPE in the form of noise |  |  |  |  |  |  |
|  |  |  | masks will be provided for |  |  |  |  |
|  |  | construction workers. | construction workers. |  |  |  |  |
|  |  |  | *Use of water sprays to |  |  |  |  |
|  |  |  | prevent dust from being air |  |  |  |  |
|  | borne |  |  |  |  |  |  |
|  |  |  | *Barricade like metal sheets |  |  |  |  |
|  |  |  | will be provided all around |  |  |  |  |
|  |  |  | the premises to avoid |  |  |  |  |
|  |  |  | fugitive dust emission into |  |  |  |  |
|  |  | the neighbouring area. | the neighbouring area. |  |  |  |  |
|  |  |  | Dust generation *PPE in the form of noise Implementation |  |  |  |  |

masks will be provided for

construction workers.

*Use

of

water

sprays

to

prevent dust from being air

borne

*Barricade like metal sheets

will be provided all around

the

premises

to

avoid

fugitive dust emission into

the neighboring area.

responsibility

–

Contractor

Water

discharge

(Construction works)

*Sewage

generated

will

be

treated in STP

Implementation

responsibility

Contractor

-

Air

emission

construction

Machinery

from

*Periodic check and regular

maintenance of construction

machinery for emissions.

*Clean

fuel

are

used

in

equipments.

Implementation

responsibility

Contractor.

–

6.2.1

Personnel Safety System:

It is planned to adapt the safe working practices which shall govern all

construction works undertaken throughout the project. The following safety aids

to all laborers will be provided.

*Safety helmets, safety Belts, Safety shoes, hand gloves.

* Gumboots while concreting

*Safety goggles while welding/stone dressing etc.

*Face mask and full body kit while pest control.

Implementation of safety procedures such as

*Using proper lifting techniques

*Using safe scaffolds

*Work permits for fabrications and welding

|  |  |  |  |  |  | masks will be provided for responsibility – |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | *Use of water sprays to |  |  |  |  |
|  |  |  |  |  | prevent dust from being air |  |  |  |  |
|  |  |  | borne |  |  |  |  |  |  |
|  |  |  |  |  | *Barricade like metal sheets |  |  |  |  |
|  |  |  |  |  | will be provided all around |  |  |  |  |
|  |  |  |  |  | the premises to avoid |  |  |  |  |
|  |  |  |  |  | fugitive dust emission into |  |  |  |  |
|  |  |  |  | the neighboring area. |  |  |  |  |  |
|  |  |  |  |  | Water discharge *Sewage generated will be Implementation |  |  |  |  |
|  |  |  |  |  | (Construction works) treated in STP responsibility - |  |  |  |  |
|  |  |  |  |  |  |  |  | Contractor |  |
|  |  |  |  |  | Air emission from maintenance of construction Implementation |  |  |  |  |
|  |  |  |  |  | construction machinery for emissions. responsibility – |  |  |  |  |
|  |  |  |  | Machinery *Clean fuel are used in Contractor. | Machinery *Clean fuel are used in Contractor. |  |  |  |  |
|  |  |  |  |  | *Periodic check and regular |  |  |  |  |
|  |  |  |  | equipments. |  |  |  |  |  |
|  |  | 6.2.1 Personnel Safety System: |  |  |  |  |  |  |  |
|  |  |  |  |  |  | It is planned to adapt the safe working practices which shall govern all |  |  |  |
|  |  |  |  |  | construction works undertaken throughout the project. The following safety aids |  |  |  |  |
|  |  | to all laborers will be provided. |  |  |  |  |  |  |  |
|  |  |  |  |  | *Safety helmets, safety Belts, Safety shoes, hand gloves. |  |  |  |  |
|  |  |  | * Gumboots while concreting |  |  |  |  |  |  |
|  |  |  |  | *Safety goggles while welding/stone dressing etc. |  |  |  |  |  |
|  |  |  |  | *Face mask and full body kit while pest control. |  |  |  |  |  |
|  |  |  | Implementation of safety procedures such as |  |  |  |  |  |  |
|  |  |  | *Using proper lifting techniques |  |  |  |  |  |  |
|  |  | *Using safe scaffolds |  |  |  |  |  |  |  |
|  |  |  |  | *Work permits for fabrications and welding |  |  |  |  |  |

6.2.2

Financial Allocation And Budgetary Provision For

EMP Aspects (Construction Aspects):

Sl. No.

Description

Financial provisions in

Lakhs

1

Environmental Management plan

1.0

0.50

during construction phase using

water for

*Sprinkling to control dusts

*Construction & curing purposes

1.0

0.50

*Flushing

0.50

0.25

2

Sewage

Treatment

plant

for

operation phase

35.0

--

3

Potable water for workers

1.0

0.50

4

Maintenance

of

vehicles

and

equipments

--

1.0

5

Temporary storm water drains

2.0

1.0

6

Personal

protection

safety

gadgets

2

0.50

7

First aid facilities for workers

1

0.50

8

Environmental

monitoring

plan

(air, noise, water and soil)

--

1.00

Total

43.50

5.75

Contingency

at 10%

4.35

0.575

Total

47.85

6.275

|  |  |  |  | 6.2.2 Financial Allocation And Budgetary Provision For |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | EMP Aspects (Construction Aspects): |  |  |  |  |
|  |  |  |  | Sl. No. Description Financial provisions in |  |  |  |
|  |  |  |  |  |  |  | Lakhs |
|  |  |  |  |  | *Sprinkling to control dusts 1.0 0.50 |  |  |
|  |  | water for |  |  |  |  |  |
|  |  |  |  |  | *Construction & curing purposes 1.0 0.50 |  |  |
|  |  |  |  |  | *Flushing 0.50 0.25 |  |  |
|  |  |  |  | 3 Potable water for workers 1.0 0.50 |  |  |  |
|  |  | equipments |  |  |  |  |  |
|  |  |  |  | 5 Temporary storm water drains 2.0 1.0 |  |  |  |
|  |  |  |  | 6 Personal protection safety 2 0.50 |  |  |  |
|  |  | gadgets |  |  |  |  |  |
|  |  |  |  | 7 First aid facilities for workers 1 0.50 |  |  |  |
|  |  |  | (air, noise, water and soil) |  |  |  |  |
| Total | Total |  |  |  |  |  |  |
|  |  |  |  | 43.50 5.75 |  |  |  |
|  |  |  | Contingency 4.35 0.575 |  |  |  |  |
| at 10% | at 10% |  |  |  |  |  |  |
|  |  |  | Total 47.85 6.275 |  |  |  |  |

6.3

3: EMP during Operation Phase:

Following

are

the

identified

operational

phase

activities

in

the

impact

assessment which may have impact on the environment.

1)

Air quality

2)

Water quality

3)

Noise quality

4)

Solid waste disposal

5)

Green belt development

6)

Storm water management

6.3.1Air Quality Management:

The pollutants envisaged from the proposed project are SPM, Nox, HC

and CO mainly due to burning of liquid fuel (HSD) in DG sets. Exhaust

from DG sets will be emitted from stack of adequate height for dispersion of

gaseous pollutants. The following table presents the EMP for air quality

management during operation phase.

Air quality management during operation phase:

Environmental

Impacts

Mitigation Proposed

D.G. Set

Ambient air quality

*Equipment

selected

will

ensure

the

exhaust

emission standard as prescribed as per the latest

amendments from MOEF

*DG will be used as standby unit

*Periodic check and maintenance

*Ambient

air

quality

monitoring

as

per

the

prescribed norms at regular interval

6.3.2

: Water Quality Management:

Water requirement of project will be augmented through KWA source

Details of water requirement and water balance is presented in chapter 2.4.

The sewage generated from the project will be treated in the proposed

STP of total capacity 30 KLD.

The treatment scheme for domestic sewage

|  |  |  | 6.3 3: EMP during Operation Phase: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Following are the identified operational phase activities in the impact |  |  |  |  |  |
|  |  |  | assessment which may have impact on the environment. |  |  |  |  |  |  |  |
|  | 1) Air quality |  |  |  |  |  |  |  |  |  |
|  | 2) Water quality |  |  |  |  |  |  |  |  |  |
|  | 3) Noise quality |  |  |  |  |  |  |  |  |  |
|  | 4) Solid waste disposal | 4) Solid waste disposal |  |  |  |  |  |  |  |  |
|  |  | 5) Green belt development |  |  |  |  |  |  |  |  |
|  |  | 6) Storm water management |  |  |  |  |  |  |  |  |
|  |  | 6.3.1Air Quality Management: |  |  |  |  |  |  |  |  |
|  |  |  |  |  | The pollutants envisaged from the proposed project are SPM, Nox, HC |  |  |  |  |  |
|  |  |  |  |  | and CO mainly due to burning of liquid fuel (HSD) in DG sets. Exhaust |  |  |  |  |  |
|  |  |  |  |  | from DG sets will be emitted from stack of adequate height for dispersion of |  |  |  |  |  |
|  |  |  |  |  | gaseous pollutants. The following table presents the EMP for air quality |  |  |  |  |  |
|  |  | management during operation phase. | management during operation phase. |  |  |  |  |  |  |  |
|  |  |  |  | Environmental Mitigation Proposed |  |  |  |  |  |  |
|  | Impacts | Impacts |  |  |  |  |  |  |  |  |
|  |  |  |  |  | D.G. Set *Equipment selected will ensure the exhaust |  |  |  |  |  |
|  |  |  |  |  | Ambient air quality *Ambient air quality monitoring as per the |  |  |  |  |  |
|  |  |  |  |  |  |  | emission standard as prescribed as per the latest |  |  |  |
|  |  |  |  |  | amendments from MOEF |  |  |  |  |  |
|  |  |  |  |  |  | *DG will be used as standby unit |  |  |  |  |
|  |  |  |  |  |  | *Periodic check and maintenance |  |  |  |  |
|  |  |  |  |  |  | prescribed norms at regular interval |  |  |  |  |
|  |  | 6.3.2 : Water Quality Management: |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Water requirement of project will be augmented through KWA source |  |  |  |  |  |
|  |  |  |  | Details of water requirement and water balance is presented in chapter 2.4. |  |  |  |  |  |  |
|  |  |  |  |  | The sewage generated from the project will be treated in the proposed |  |  |  |  |  |
|  |  |  |  |  | STP of total capacity 30 KLD. The treatment scheme for domestic sewage |  |  |  |  |  |

generated from project is discussed in chapter 2.8. Treated water will be reused

for flushing, gardening etc. and the following table presents the EMP for water

quality.

Water Quality Management during operation phase

:

Environmental impacts

Mitigation proposed

Waste water

*Treated with proposed sewage Treatment plant

to produce tertiary treated water which is used for

secondary

purposes

such

as

flushing,

and

landscaping

*Water

conservation

measures

will

be

encouraged.

6.3.3

: Noise Management:

High Noise generating units such as DG sets will be provided with acoustic

enclosures. Green belt on the project boundary will further act as noise barrier.

The table presents the EMP for noise level.

Noise management during operation phase

:

6.3.4

Solid Waste Management:

The solid wastes generated during operation phase can be categorized as:

*Wet garbage

: Food waste

*Dry garbage

: Paper, plastic, bottles etc.

*Sludge from sewage treatment plant (STP)

The various mitigation measures to be adopted during collection and disposal of

wastes are as follows:

Noise from DG set area

*Acoustic enclosures will be provided for DG set

*DG set will be installed in an area where the

access will be restricted.

*The use of PPE (earplugs) will be mandatory in

this area.

*Selection of equipment to ensure that the residual

noise level of ,<55dB(A)

*Noise levels will be checked periodically using a

noise pressure level meter.

|  |  |  |  |  | generated from project is discussed in chapter 2.8. Treated water will be reused |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | for flushing, gardening etc. and the following table presents the EMP for water |  |  |  |  |
| quality. |  |  |  |  |  |  |  |  |  |
|  |  |  | Water Quality Management during operation phase : |  |  |  |  |  |  |
|  |  |  | Environmental impacts Mitigation proposed |  |  |  |  |  |  |
|  |  |  |  |  | Waste water *Treated with proposed sewage Treatment plant |  |  |  |  |
|  |  |  |  |  |  | to produce tertiary treated water which is used for |  |  |  |
|  |  |  |  |  |  | secondary purposes such as flushing, and |  |  |  |
|  |  |  |  | landscaping |  |  |  |  |  |
|  |  |  |  |  |  | *Water conservation measures will be |  |  |  |
|  |  |  |  | encouraged. |  |  |  |  |  |
|  | 6.3.3 : Noise Management: |  |  |  |  |  |  |  |  |
|  |  |  |  |  | High Noise generating units such as DG sets will be provided with acoustic |  |  |  |  |
|  |  |  |  |  | enclosures. Green belt on the project boundary will further act as noise barrier. |  |  |  |  |
|  |  | The table presents the EMP for noise level. |  |  |  |  |  |  |  |
|  |  | Noise management during operation phase : |  |  |  |  |  |  |  |
|  |  | 6.3.4 Solid Waste Management: |  |  |  |  |  |  |  |
|  |  |  |  | The solid wastes generated during operation phase can be categorized as: |  |  |  |  |  |
|  |  | *Wet garbage : Food waste |  |  |  |  |  |  |  |
|  |  |  | *Dry garbage : Paper, plastic, bottles etc. |  |  |  |  |  |  |
|  |  |  | *Sludge from sewage treatment plant (STP) |  |  |  |  |  |  |
|  |  |  |  |  | The various mitigation measures to be adopted during collection and disposal of |  |  |  |  |
|  | wastes are as follows: |  |  |  |  |  |  |  |  |
|  | Noise from DG set area |  |  |  | access will be restricted. |  |  |  |  |
|  |  |  |  |  |  | *Acoustic enclosures will be provided for DG set |  |  |  |
|  |  |  |  |  |  | *DG set will be installed in an area where the |  |  |  |
|  |  |  |  |  |  | *The use of PPE (earplugs) will be mandatory in |  |  |  |
|  |  |  |  | this area. |  |  |  |  |  |
|  |  |  |  |  |  | *Selection of equipment to ensure that the residual |  |  |  |
|  |  |  |  |  | noise level of ,<55dB(A) |  |  |  |  |
|  |  |  |  |  |  | *Noise levels will be checked periodically using a |  |  |  |
|  |  |  |  |  | noise pressure level meter. |  |  |  |  |

*It is preferable that the container and bins used for collection of waste

should be of closed type and waste is not exposed thus possibility spreading of

disease through flies and mosquitoes is minimized.

*Collection system should be properly supervised so that quick and regular

removal of waste from the dustbin is practiced.

6.4

4: Health Risk And Disaster Management:

6.4.1

: Public and Health Safety:

Since all the construction related activities are confined to the project site,

minimal health related impacts are envisaged within the project influenced area

during the construction stage.

At the project site on an average of 300 No. of persons will be engaged,

who face direct exposure to dust and noise generated from the construction

activity. This may likely to cause health related problem such

as asthma,

bronchitis etc.

To minimize these anticipated impacts, suitable actions like

1.

Use of water sprinklers to prevent dust from being airborne.

2.

Providing suitable PPE kit like mask with filter, helmets etc.

3.

Periodic health check-up camp for the laborers will be arranged

4.

Provision of safety belts.

5.

In

case

of

injury

on

site

medical

treatment

and

transport

will

be

organized.

6.4.2

Risk and Disaster Management Plan:

Construction phase:

Sl.

No.

Potential

Mitigation

1

Accidental fire

Fire safety gadgets

2

Fall of objects

Use of personnel protection devises-helmets

3

Working at great

heights

Protection to prevent fall, with life safety belts

and nets

4

Accident

from

Machinery

Personnel protection gadgets

5

Electrical mishap

Adopting safety measures to prevent any act of

negligence

and

providing

electrical

safety

measures like fire extinguishers.

|  |  |  |  |  |  | *It is preferable that the container and bins used for collection of waste |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | should be of closed type and waste is not exposed thus possibility spreading of |  |  |  |  |
|  |  |  | disease through flies and mosquitoes is minimized. |  |  |  |  |  |  |
|  |  |  |  |  |  | *Collection system should be properly supervised so that quick and regular |  |  |  |
|  |  |  | removal of waste from the dustbin is practiced. |  |  |  |  |  |  |
|  |  |  |  | 6.4 4: Health Risk And Disaster Management: |  |  |  |  |  |
|  |  | 6.4.1 : Public and Health Safety: |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Since all the construction related activities are confined to the project site, |  |  |  |
|  |  |  |  |  | minimal health related impacts are envisaged within the project influenced area |  |  |  |  |
|  | during the construction stage. | during the construction stage. |  |  |  |  |  |  |  |
|  |  |  |  |  |  | At the project site on an average of 300 No. of persons will be engaged, |  |  |  |
|  |  |  |  |  | who face direct exposure to dust and noise generated from the construction |  |  |  |  |
|  |  |  |  |  | activity. This may likely to cause health related problem such as asthma, |  |  |  |  |
|  |  |  |  |  | To minimize these anticipated impacts, suitable actions like |  |  |  |  |
|  |  |  |  | 1. Use of water sprinklers to prevent dust from being airborne. | 1. Use of water sprinklers to prevent dust from being airborne. |  |  |  |  |
|  |  |  |  | 2. Providing suitable PPE kit like mask with filter, helmets etc. | 2. Providing suitable PPE kit like mask with filter, helmets etc. |  |  |  |  |
|  |  |  |  |  | 3. Periodic health check-up camp for the laborers will be arranged |  |  |  |  |
|  |  | 4. Provision of safety belts. |  |  |  |  |  |  |  |
|  |  |  |  |  | 5. In case of injury on site medical treatment and transport will be | 5. In case of injury on site medical treatment and transport will be |  |  |  |
|  | organized. |  |  |  |  |  |  |  |  |
|  |  |  | 6.4.2 Risk and Disaster Management Plan: |  |  |  |  |  |  |
|  |  | Construction phase: |  |  |  |  |  |  |  |
|  |  |  |  | Sl. Potential Mitigation |  |  |  |  |  |
|  |  |  | 1 Accidental fire Fire safety gadgets |  |  |  |  |  |  |
|  |  |  |  |  | 2 Fall of objects Use of personnel protection devises-helmets |  |  |  |  |
|  |  |  |  |  | 3 Working at great Protection to prevent fall, with life safety belts |  |  |  |  |
|  |  |  | heights and nets |  |  |  |  |  |  |
|  |  |  |  | 4 Accident from Personnel protection gadgets |  |  |  |  |  |
|  |  | Machinery |  |  |  |  |  |  |  |
|  |  |  |  |  | 5 Electrical mishap Adopting safety measures to prevent any act of |  |  |  |  |
|  |  |  |  |  |  |  | negligence and providing electrical safety |  |  |
|  |  |  |  |  |  | measures like fire extinguishers. |  |  |  |

Operation phase:

Disaster is an unexpected event due to sudden failure of the system, external

threats, internal disturbances, fire and accidents.

An appropriate management plan shall be incorporated.

Precautions:

•

Once the likelihood of the disaster is suspected, preventive actions should be

undertaken by the project-in-charge.

•

Conditional maintenance of equipments, materials and expertise for use

during emergency.

•

The electrical system shall be provided with automatic circuit breakers

activated by over current.

•

Proper escape routes are planned and displayed in the public domain.

•

Selected

representatives

are

given

proper

training

to

guide

inhabitants

during fire accidents.

•

Periodic awareness program is conducted to the workers on their roles

during emergency situations.

•

Important telephone numbers like police, Fire Department and hospitals for

use during emergency situations will be made available.

6.5

5: EMP Implementation Schedule:

Sl.

No.

Recommendations

Requirements

1

Air

pollution

Control

measures

Before commissioning of respective units.

2

Water pollution control

measures

Before commissioning of the project

3

Noise control measures

Along

with

the

commissioning

of

the

project

4

Solid

waste

management

During commissioning of the project

5

Green belt development

Stage-wise implementation

|  |  | Operation phase: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Disaster is an unexpected event due to sudden failure of the system, external |  |  |  |  |
|  |  |  | threats, internal disturbances, fire and accidents. | threats, internal disturbances, fire and accidents. |  |  |  |  |  |
|  |  |  |  | An appropriate management plan shall be incorporated. |  |  |  |  |  |
|  | Precautions: |  |  |  |  |  |  |  |  |
|  |  |  |  |  | • Once the likelihood of the disaster is suspected, preventive actions should be |  |  |  |  |
|  |  |  | undertaken by the project-in-charge. |  |  |  |  |  |  |
|  |  |  |  |  | • Conditional maintenance of equipments, materials and expertise for use |  |  |  |  |
|  |  | during emergency. |  |  |  |  |  |  |  |
|  |  |  |  |  | • The electrical system shall be provided with automatic circuit breakers |  |  |  |  |
|  |  | activated by over current. | activated by over current. |  |  |  |  |  |  |
|  |  |  |  |  | • Proper escape routes are planned and displayed in the public domain. |  |  |  |  |
|  |  |  |  |  | • Selected representatives are given proper training to guide inhabitants |  |  |  |  |
|  |  | during fire accidents. |  |  |  |  |  |  |  |
|  |  |  |  |  | • Periodic awareness program is conducted to the workers on their roles |  |  |  |  |
|  |  |  | during emergency situations. |  |  |  |  |  |  |
|  |  |  |  |  | • Important telephone numbers like police, Fire Department and hospitals for |  |  |  |  |
|  |  |  |  | 6.5 5: EMP Implementation Schedule: |  |  |  |  |  |
|  |  |  | Sl. Recommendations Requirements | Sl. Recommendations Requirements |  |  |  |  |  |
| No. |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1 Air pollution Control Before commissioning of respective units. |  |  |  |  |
|  |  | measures |  |  |  |  |  |  |  |
|  |  |  |  |  | 2 Water pollution control Before commissioning of the project |  |  |  |  |
|  |  | measures |  |  |  |  |  |  |  |
|  |  |  |  |  | 3 Noise control measures Along with the commissioning of the |  |  |  |  |
|  |  |  |  |  | project |  |  |  |  |
|  |  |  |  |  | 4 Solid waste During commissioning of the project |  |  |  |  |
|  |  | management |  |  |  |  |  |  |  |
|  |  |  |  | 5 Green belt development Stage-wise implementation |  |  |  |  |  |

Financial Allocation and Budgetary provision for EMP aspects (Occupancy

phase)

Sl.

No.

Description

Financial Provision In

Lakhs

Capital

cost

Recurring

cost

1

Operation of sewage treatment plant

--

2.0

2

Rain water harvesting tanks and its facilities

10.0

2.0

3

Ground

water

recharging

pits

&

its

management

5.0

1.0

4

DG sets acoustic maintenance

5.00

1.0

5

Landscaping

4.0

1.0

6

Solid waste management

4.0

1.0

7

Environmental Monitoring Plan per annum

--

1.0

Total

28.0

9.0

Contingency at 10%

2.8

9.0

Total

30.80

9.90

A comprehensive monitoring program is suggested below

6.6

6: Environmental Monitoring Routines:

A comprehensive monitoring program is suggested below

Sl.

No.

Particulars

Monitoring

Frequency

Duration Of

Monitories

Important

Parameters For

Monitoring

1

Air Quality

2

Ambient Air Monitories

Project premises

Once

in

a

month

24

hourly

sample

RSPM, SPM, SOZ,

NOX

2

Stack monitoring

Once

in

a

year

if

required

Grab

SPM,

SOZ,

NOX,

HC, CO

II

Water and Waste water quality

1

Water Quality

a)

Ground Water at two locations

of treated effluent discharge are

Ones in a

month

Grab

As per PCB

requirement

2

Waste water quality

|  |  |  |  |  | Financial Allocation and Budgetary provision for EMP aspects (Occupancy |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| phase) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Sl. Description Financial Provision In |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Capital Recurring |  |
|  |  |  |  |  |  |  |  | cost cost |  |  |
|  |  |  | 3 Ground water recharging pits & its |  |  |  |  |  |  |  |
|  |  |  |  |  | management 5.0 1.0 |  |  |  |  |  |
|  |  |  |  |  | Total 28.0 9.0 | Total 28.0 9.0 |  |  |  |  |
|  |  |  |  |  | Contingency at 10% 2.8 9.0 |  |  |  |  |  |
|  |  |  |  |  | Total 30.80 9.90 |  |  |  |  |  |
|  |  |  | 6.6 6: Environmental Monitoring Routines: | 6.6 6: Environmental Monitoring Routines: |  |  |  |  |  |  |
|  |  |  |  |  | No. Frequency Monitories Parameters For |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Monitoring |  |
|  | 1 Air Quality |  |  |  |  |  |  |  |  |  |
|  |  | 2 Ambient Air Monitories |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Project premises Once in a 24 hourly RSPM, SPM, SOZ, | Project premises Once in a 24 hourly RSPM, SPM, SOZ, |  |  |  |  |
|  |  |  |  |  |  | month sample NOX |  |  |  |  |
|  |  |  |  |  | 2 Stack monitoring Once in a Grab SPM, SOZ, NOX, |  |  |  |  |  |
|  |  |  |  |  |  | year if HC, CO |  |  |  |  |
|  |  |  |  | required |  |  |  |  |  |  |
|  |  | II Water and Waste water quality |  |  |  |  |  |  |  |  |
|  | 1 Water Quality |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | a) Ground Water at two locations Ones in a Grab As per PCB |  |  |  |  |  |
|  |  |  |  |  | of treated effluent discharge are month requirement | of treated effluent discharge are month requirement |  |  |  |  |
|  | 2 Waste water quality |  |  |  |  |  |  |  |  |  |

a)

Inlet into STP

N.A.

N.A.

b)

Treated

effluent

prior

to

discharge

N.A.

N.A.

III

Soil Quality

1

Within project premises at

1

location

on

effluent

discharging area

Ones

in

6 months

Composite

sample

As

per

PCB

requirements

2

Ecological preservation and

up-gradation

Seasonal

Visual

observation

Survival rate

IV

Noise Monitoring

1

Project premises

Ones in 6

months

Day

and

night

As

per

PCB

requirements

6.7

7: Traffic Management Measures:

•

Merging of vehicles will be performed only to leave vehicle from the

gate, this ensues safety.

•

To

establish

smooth

entry

and

exit

of

vehicles,

bell

mouth

shape

geometry is provided at the gates.

•

Yellow

paint

junction

boxes

are

painted

at

the

locations

to

create

psychological barrier for through drivers to control the speed.

•

Adequate sign & guide posts for traffic as per IRC:

•

Road marking, stop lines, parking lines, slot numbers etc. must be clearly

painted so as to guide the vehicles.

6.8

8: Energy Conservation Aspects Proposed In The Project:

•

Energy efficient motors, whose efficiency is not lower than the limits

specified in section 8.2.2 of the energy conservation building code will be

installed.

•

Dry type transformer with efficiency not lower than the limits specified in

section 8.2.1 of the code shall be considered.

•

Power monitoring, recording and check metering will be provided in

main LT panels on the incoming feeders and individual outgoing feeders

covering all parameters such as current, voltage, and energy as stipulated

in section 8.2. 4 of the Code.

•

Cable size and design of distribution system will ensure that the total

distribution loss will not exceed 1% of the total energy consumed as

stipulated in section 8.2.5.1 of the Code.

|  |  |  |  | a) Inlet into STP N.A. N.A. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | b) Treated effluent prior to N.A. N.A. |  |  |  |  |  |  |
|  |  | discharge |  |  |  |  |  |  |  |  |
|  | III Soil Quality |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1 Within project premises at Ones in Composite As per PCB |  |  |  |  |  |
|  |  |  |  |  |  | 1 location on effluent 6 months sample requirements |  |  |  |  |
|  |  | discharging area |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 2 Ecological preservation and Seasonal Visual Survival rate |  |  |  |  |  |
|  |  |  |  |  | up-gradation observation |  |  |  |  |  |
|  |  | IV Noise Monitoring |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1 Project premises Ones in 6 Day and As per PCB |  |  |  |  |  |
|  |  |  |  |  |  |  |  | months night requirements |  |  |
|  |  |  | 6.7 7: Traffic Management Measures: |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Merging of vehicles will be performed only to leave vehicle from the |  |  |  |  |
|  |  | gate, this ensues safety. | gate, this ensues safety. |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • To establish smooth entry and exit of vehicles, bell mouth shape |  |  |  |  |
|  |  |  | geometry is provided at the gates. |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Yellow paint junction boxes are painted at the locations to create |  |  |  |  |
|  |  |  |  |  | psychological barrier for through drivers to control the speed. |  |  |  |  |  |
|  |  |  |  | • Adequate sign & guide posts for traffic as per IRC: |  |  |  |  |  |  |
|  |  |  |  |  |  | • Road marking, stop lines, parking lines, slot numbers etc. must be clearly |  |  |  |  |
|  |  |  | painted so as to guide the vehicles. |  |  |  |  |  |  |  |
|  |  |  |  |  | 6.8 8: Energy Conservation Aspects Proposed In The Project: |  |  |  |  |  |
|  |  |  |  |  |  | • Energy efficient motors, whose efficiency is not lower than the limits |  |  |  |  |
|  |  |  |  |  |  | specified in section 8.2.2 of the energy conservation building code will be |  |  |  |  |
|  | installed. | installed. |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Dry type transformer with efficiency not lower than the limits specified in |  |  |  |  |
|  |  |  |  | section 8.2.1 of the code shall be considered. |  |  |  |  |  |  |
|  |  |  |  |  |  | • Power monitoring, recording and check metering will be provided in |  |  |  |  |
|  |  |  |  |  |  | main LT panels on the incoming feeders and individual outgoing feeders |  |  |  |  |
|  |  |  |  |  |  | covering all parameters such as current, voltage, and energy as stipulated |  |  |  |  |
|  |  |  | in section 8.2. 4 of the Code. |  |  |  |  |  |  |  |
|  |  |  |  |  |  | • Cable size and design of distribution system will ensure that the total |  |  |  |  |
|  |  |  |  |  |  | distribution loss will not exceed 1% of the total energy consumed as |  |  |  |  |
|  |  |  |  | stipulated in section 8.2.5.1 of the Code. |  |  |  |  |  |  |

•

Timer controls will be incorporated in the distribution board of external

lighting to save energy.

•

Solar lighting shall be provided at strategic locations in outdoor areas,

•

Water cooled chillers are used instead of air-cooled chillers to save

energy.

•

VFD driver shall be provided for secondary pumps for water cooled

chillers and lifts which will result in energy savings.

|  |  |  |  | • Timer controls will be incorporated in the distribution board of external |
| --- | --- | --- | --- | --- |
|  |  | lighting to save energy. |  |  |
|  |  |  |  | • Solar lighting shall be provided at strategic locations in outdoor areas, |
|  |  |  |  | • Water cooled chillers are used instead of air-cooled chillers to save |
|  | energy. |  |  |  |
|  |  |  |  | • VFD driver shall be provided for secondary pumps for water cooled |
|  |  |  | chillers and lifts which will result in energy savings. |  |